The PeakStream Platform:

High-Productivity Software Development for Multi-Core Processors

Matthew Papakipos
PeakStream, Inc.

April 10, 2007

Abstract

This paper discusses the PeakStream Platform, a new software development platform that offers an easy-to-use stream programming model for multi-core processors and accelerators such as graphics processing units (GPUs). Although accelerators such as GPUs can provide dramatic performance advantages for high-performance computing (HPC) applications, they can also present significant challenges for application developers. The PeakStream Platform overcomes those challenges by offering a developer-friendly and efficient interface.

This paper describes the application view of the PeakStream Platform and its solutions to the challenges of multi-core and accelerator programming. It provides application code samples and comparisons between stream programming and traditional serial programming.

This information applies for the following operating systems:
	Windows Server® Code Name “Longhorn”
	Windows Vista™
	Microsoft® Windows Server 2003
	Microsoft Windows® XP
	Microsoft Windows 2000

Contents

1 Introduction	3

1.1 Overview	3

2 Multi-Core Processors	4

2.1 Multi-Core CPUs	4

2.2 GPUs as Accelerators	5

2.3 GPU Programming Challenges	6

3 The PeakStream Platform	8

3.1 Structure of PeakStream Platform Applications	9

3.2 Array Data Types and Operations	9

3.3 Just-in-Time/Dynamic Translation	10

3.4 PeakStream Platform Headers and Libraries	10

3.5 C++ Interfaces	11

3.6 PeakStream Platform Tools	11

4 Solutions to GPU Issues	12

4.1 Software Stability when Processor Architecture Changes	12

4.2 Tool Support and Compatibility	13

4.3 Accurate Mathematical Library Support	13

4.4 Computationally Intense GPU Kernels	13

4.5 Progressive Evaluation	14

4.6 I/O Cost Analysis	14

5 Examples of Using the PeakStream APIs	15

5.1 Monte Carlo Options Pricing Source Code	15

5.2 Monte Carlo Options Pricing Using Stream Programming	16

6 Related Work	16

6.1 Commercial GPU Shader Languages	17

6.2 GPU Software Research Efforts	17

7 PeakStream: The New Generation of Multi-Core Systems	18

References	18

Windows Hardware Engineering Conference - WinHEC Sponsors’ Disclaimer:  The contents of this document have not been authored or confirmed by Microsoft or the WinHEC conference co-sponsors (hereinafter “WinHEC Sponsors”). Accordingly, the information contained in this document does not necessarily represent the views of the WinHEC Sponsors and the WinHEC Sponsors cannot make any representation concerning its accuracy. THE WinHEC SPONSORS MAKE NO WARRANTIES, EXPRESS OR IMPLIED, WITH RESPECT TO THIS INFORMATION.

©COPYRIGHT 2006-2007 by PeakStream, Inc. All Rights Reserved. Patents pending. PeakStream and Progressive Evaluation are trademarks of PeakStream Inc. in the United States and/or other jurisdictions. All other marks and names mentioned herein may be trademarks of their respective companies.

Microsoft, Direct3D, DirectX, Visual Studio, Windows, Windows Server, and Windows Vista are either registered trademarks or trademarks of Microsoft Corporation in the United States and/or other countries. The names of actual companies and products mentioned herein may be the trademarks of their respective owners.

# 1 Introduction

Since the beginning of modern computing, most computer programs have been written by using a serial programming model. Apart from a brief era of parallel programming in the late 1980s on systems from companies such as Thinking Machines and MasPAR, serial programming has been the predominant programming model for more than 50 years. But with the availability of new, powerful data parallel accelerators, such as the graphics processing unit (GPU), as well as the imminent arrival of highly multi-core CPUs (the recently introduced Intel Polaris prototype has 80 cores), the serial programming model is running into foreseeable and crippling limitations. The emerging world of highly parallel systems requires a programming model that scales to this new generation of parallel architectures.

The PeakStream Platform embodies a new parallel programming model that is called “stream programming.” PeakStream solves the classic challenges of parallel programming by using data arrays for its core objects and makes the task of distributing work across multiple cores tractable.

The PeakStream Platform is designed for computationally intensive applications and provides an easy-to-use interface abstraction that is efficient, addresses the implementation details of various parallel architectures transparently to the application developer, and enables application portability between these architectures. A PeakStream program written to be accelerated with today’s GPUs, for example, will run seamlessly on the highly multi-core CPUs of tomorrow, without rewriting or recompilation.

Developers can take full advantage of multi-core systems and GPU-based acceleration by using the PeakStream C and C++ APIs. Those APIs are implemented by libraries that dynamically translate API calls into parallel programs and execute them. These parallel programs are optimized to generate fast, numerically accurate code for the target platform. The PeakStream Platform also includes debugging support and profiling tools. In addition, it has been designed to interoperate well within a developers’ existing software development environment, including both Linux and Window development applications. In summary, the PeakStream Platform is designed for computationally intensive applications that want to take advantage of the impressive performance of the new generation of multi-core systems and gain easy access to the impressive acceleration potential of GPUs.

## 1.1 Overview

The performance of multi-core processors, as well as the set of issues that makes them difficult to program, is discussed in Section 2, with particular reference to using GPUs as accelerators.

The PeakStream Platform is introduced and examined in three sections. In Section 3, an application-level view of the platform is provided, along with a discussion of some operational details. Section 4 discusses the ways that the PeakStream Platform addresses the challenges of multi-core programming. A larger code sample is shown in Section 5, and its performance is compared to that of a serial implementation of the same algorithm.

Section 6 relates the PeakStream Platform to other work that has been done to enable higher efficiency and higher productivity programming on multi-core processors.

Finally, Section 7 concludes the paper.

# 2 Multi-Core Processors

Traditional “single-core” CPU performance hit a fundamental performance limitation in early 2000 due to power and implementation difficulties. Until this wall was reached, applications could move seamlessly from one generation of processor to the next, taking advantage of the increased clock speed and cache sizes without needing to change anything in their code. Now, however, the major CPU manufacturers have shifted to a multi-core strategy. High-end commodity CPUs today are quad-core x86 processors. By the end of 2008, the first oct-core processors will ship, and by 2010, systems with more than 32 cores are projected. Clock speeds have been essentially stagnant for more than four years and are not projected to increase for the remainder of the decade. In many cases, clock speeds are actually declining due to the increased market demand for lower power consumption.

Although CPUs have only recently become multi-core, other computing platforms have been multi-core for quite some time. Specialized digital signal ASICs have been highly multi-core for many years, and due to the inherently parallel task of rendering pixels, mainstream graphics processors have been designed as multi-core processors for well over a decade. Thanks to their higher floating point capacity, they can offer substantial performance benefits as accelerators. Today’s leading GPU offers 128 cores—the most highly multi-core commodity processor.

## 2.1 Multi-Core CPUs

The problem of programming multi-core CPUs can be explained as a problem of work distribution. Any meaningful application can be characterized as “a large number of operations on data.” In the case of a single-core CPU, the CPU steps through the operations in sequence, and there is only one actor—the single core—reading and writing data. In the case of a system with a large number of cores, however, the programmer must distribute this work across a large (and growing) number of independent cores.

To state the concepts simply, there are essentially two alternative approaches for distributing work to multiple cores. The first way—“task-parallel programming”—breaks down the list of operations into a set of reasonably independent tasks and distributes these tasks among the cores—and each core accesses the whole dataset. The second approach—“data-parallel programming”—breaks down the data into subsets of reasonably independent data and each core performs all operations on each subset.

Although there have been many programming approaches that embody every step on the continuum from task to data parallel programming, relatively few have survived as working models. Today, “multi-threaded programming” is the leading implementation of task parallel programming. And stream programming is rapidly becoming the most visible example of data parallel programming.

Multi-threaded programming has been popular for applications faced with the problem of distributing work to a relatively small number of cores. Usually most applications have at least a few independent tasks that can be assigned to different cores. With increasing core counts, however, multi-threaded programming is facing an uphill usability battle. As the number of cores increases, programmers are hard-pressed to find additional levels of task independence, and multi-threaded programs become increasingly cluttered with mechanisms to coordinate access to the shared dataset. This complex access coordination, in turn, increases nondeterminacy, complicates debugging and quality assurance, and results in nonintuitive source code whose behavior programmers cannot predict in advance. In the worst cases, this complex access coordination results in applications that unpredictably seize up permanently while multiple cores contend for data access permission.

In contrast, stream programming offers a data parallel approach that can scale to essentially unlimited numbers of cores—with datasets of sufficient size. And unlike multi-threaded programming, stream programming is far more similar to single-threaded programming in style and execution, and can be seen from PeakStream code examples later in this paper.

## 2.2 GPUs as Accelerators

Systems that leverage a GPU as an accelerator have three key architectural elements:

- An x86 CPU in its own chip package with memory attached either locally or via a “Northbridge” chip.

- A “Southbridge” chip, connecting the CPU to the GPU, such as the NVIDIA nForce 4

- A programmable GPU with locally attached memory such as the ATI R580 or NVIDIA G80.

GPU characteristics such as the size and performance of the locally attached GPU and the number of computation cores varies depending on the GPU. For example, the ATI 1950XTX data-parallel computation units have the following characteristics:

- 48 processor cores

- Each core being a four-way SIMD

- Each core capable of three single-precision flops/clock-cycle

- Compute processors operating at up to 625 MHz

- Over 50 GB/second of external memory bandwidth

Figure 1: GPU System Diagram

Figure 1 illustrates the structure of a commodity x86 system with a high-end GPU, including the bandwidths of memory and I/O buses.

### 2.2.1 Use of GPUs on HPC Algorithms

Performance of GPUs running hand-crafted code to implement high-performance computing (HPC) algorithms has been an area of substantial research. The following table shows recent results of HPC applications that have been accelerated using GPUs in a co-processor configuration.

| Performance Increases |  |
| --- | --- |
| Applications | GPU Acceleration |
| Kirchhoff Migration | 8x |
| Computed Tomography | 10x |
| Monte Carlo Simulation | 15x |
| Black Scholes Options Pricing | 15x |

## 2.3 GPU Programming Challenges

Using GPUs as application accelerators for HPC places many unique demands on the supporting development and runtime software.

### 2.3.1 Architecture changes

GPU architectures and implementations can vary significantly between vendors and across GPU generations. GPU hardware is still actively evolving, and highly optimized implementations of algorithms on GPUs from year to year are often completely different. Historically, a port to one generation of GPU has not ensured an easy port to the next generation. Consequently, the lack of portability has provided a significant barrier to entry for HPC application developers who understandably want to invest in a new application development model that can encompass multiple types of multi-core systems.

### 2.3.2 Tool support

Today, GPUs have limited tool support for developing HPC applications. In particular, application-level profilers and debuggers do not exist.

For graphics programming in Microsoft DirectX® 9, Microsoft provides support for a debugger [Microsoft06] that can allow setting breakpoints and examining data in simulated GPU shaders. These simulated shaders operate entirely on the CPU, and simulate GPU shader computations. HPC algorithms (such as FFT) can easily translate into 20 or more GPU shaders for an algorithm. In addition, it does not expose performance or numerical issues that might only appear when running on real GPU hardware. Thus, debugging with simulated shader programs is inappropriate for HPC application developers.

GPUs do provide hardware support for performance monitoring, and some tools to access that information [Domine05]. Common GPU developer tools provide a visual display of performance information while rendering graphics application frames for interactive display. Unfortunately, they are directed at graphics applications rather than HPC applications. In particular, they require the use of OpenGL or DirectX, which is inappropriate for compute-based applications that have no interactive graphical display.

### 2.3.3 Arithmetic issues

GPU math libraries have been developed for use by interactive graphics applications and in many cases suffer from substantial accuracy errors in basic math operations. For graphics applications, these accuracy errors are not noticeable since they result in transient single-pixel errors on the real-time display, effectively appearing as white noise. But for HPC applications, these errors are unacceptable since they jeopardize the quality of the underlying numerical simulation. In Figure 2, we present findings about exp accuracy on a modern GPU. Error is shown relative to a CPU implementation using IEEE 32-bit (single) floating point arithmetic. In the PeakStream VM, we provide an exp() function with more reasonable error characteristics for HPC applications.

Figure 2: Accuracy of exp(x)

### 2.3.4 Creating Suitable Compute Kernels

To realize the performance potential of GPUs, it is critical to ensure that performance does not become memory-bandwidth constrained. When examining the flop/memory-access ratio (also called computational intensity [Brook04]), of GPUs, GPUs have more floating-point units available for computation-per-memory-access than developers are used to on CPUs.

Having a high ratio of FLOP/memory access is both a blessing and a curse. On the positive side, this provides high peak computational performance. On the negative side, it becomes critical on GPUs to structure computation in a way that ensures all these FLOPS are used effectively.

The following table computes the ideal FLOP/memory-access ratio for the ATI R580 for a simple operation with one input array and one output array:

| Memory Comparison |  |  |
| --- | --- | --- |
| Compute performance | 375 | GFlop/s |
| Memory bandwidth | 53 | GBytes/s |
| Compute/memory transfer | 7 | flops/(read+written bytes) |
| Memory transfers/kernel | 8 | Bytes (2 floats) read+written/kernel |
| Compute/kernel | 58 | flops/kernel |

Thus, for the ATI R580 GPU, for a simple operation with one input and one output, the ideal computational intensity (assuming a linear memory access pattern) is 58. Kernels with ratios lower than this will have performance that is memory limited. Kernels with ratios higher than this will be compute limited. In general, the goal in developing kernels is to maximize the computational intensity. Naively coded kernels, by contrast, tend to be memory limited.

For a simple unary element-wise operation, like –x, high computational intensity is unachievable. The way to generate dense computational kernels is to exploit producer-consumer locality and combine back-to-back element-wise operations. Exploitation of producer-consumer locality is one of the key characteristics of efficient algorithms for multi-core processors [Dally03].

### 2.3.5 Loosely Coupled Processor

The data-parallel computation units of a GPU operate asynchronously from the operation of the CPU with which they are coupled. For graphics applications, the GPU typically runs about one frame of latency behind the CPU. For an interactive visualization application, this means about 16 milliseconds of latency (16 msec = 1/60 of a second, which is a typical real-time rate of display). There are good reasons for GPUs to behave this way. In particular, it allows temporal load balancing of the command queue between the CPU and GPU. Graphics applications running on the CPU are typically very bursty, so the large (16-msec) command queue is included to avoid having the GPU “run dry” during periods when the CPU is not actively feeding commands to the GPU [Akeley93].

#### 2.3.5.1 High Latency

The PeakStream Platform views the GPU as a loosely coupled, high-latency co-processor. For good performance, the CPU must send a significant queue of work to the GPU for execution and write it in a way that is latency tolerant. The requirement for large work transfers has led to the PeakStream design decision to disallow direct CPU access to data structures on the compute units on the GPU. Doing so would require that the CPU and GPU operate in tight synchrony, which would destroy the performance advantages of the GPU.

#### 2.3.5.2 High CPU/GPU Synchronization Costs

Reading data from the GPU back to the CPU is expensive. It requires that the GPU complete the computations that are queued for its execution, which can often take tens of milliseconds, depending on how many commands are queued up for the GPU. Shortening the GPU command queue does not help because it just creates CPU/GPU temporal load balancing problems, as previously described. Synchronizing the CPU and GPU also has the undesirable consequence of forcing the GPU to momentarily “run dry,” creating a window where the computational cycles on the GPU are completely wasted, until the CPU can queue some future commands for it.

To address this issue, the PeakStream Platform is structured so that an entire chain of computations can be moved to the GPU. By moving these computations to the GPU, the need to move data frequently between the processors and incur synchronization costs is reduced.

# 3 The PeakStream Platform

The PeakStream Platform is a comprehensive data parallel application development platform designed to unlock the performance potential of multi-core processors and allow the easy use of accelerators such as GPUs. The platform consists of four major components: the PeakStream APIs, the PeakStream VM, the PeakStream Profiler, and the PeakStream Debugger.

## 3.1 Structure of PeakStream Platform Applications

Figure 3 shows the structure of an application interacting with the PeakStream Platform.

Figure 3: Application using the PeakStream Platform

Applications are coded to use the PeakStream APIs and are linked against the PeakStream Virtual Machine libraries. The libraries handle all of the details of interaction with the processor. When the application uses the PeakStream APIs to perform mathematical operations (such as addition or use of math library functions like exp), those API calls are processed by the VM. The VM then creates optimized parallel kernels that are executed on the processor. The application must use explicit I/O calls (read and write) to move data into and out of the VM. It is important to stress that compute kernels are dynamically synthesized at runtime. Because target processors may have widely divergent architectures, it is important to make kernel density and boundary decisions at runtime rather than hard-coding them at application design time.

## 3.2 Array Data Types and Operations

The primary data type presented to the application developer by the PeakStream APIs is the Array. Arrays come in several flavors:

- Arrayf32: dense 32-bit (IEEE single) floating point elements.

- Arrayf64: dense 64-bit (IEEE double) floating point elements.

Arrays can represent scalar (1x1), vector (Nx1), and two-dimensional or matrix (MxN) data. All operations provided by the PeakStream APIs operate on arrays. Static type checking is performed based on array type and dynamic checking, and error reporting is used to signal errors such as array size mismatches or use of invalid array values.

All of the basic arithmetic operations (such as addition and multiplication) can be performed on arrays. In most cases, arithmetic operations are performed element-wise, that is, the corresponding elements of two arrays will be added or multiplied to produce a result array that has the same size as the inputs. Scalars are implicitly converted to 1-D and 2-D arrays for use in element-wise operations, yielding semantics that are similar to those provided by Mathworks Matlab and Fortran 90.

In addition to the basic arithmetic operations, PeakStream provides a substantial math library. It includes transcendental and trigonometric functions that operate on arrays, as well as matrix and vector reduction and manipulation functions. Advanced intrinsic functions including matrix multiplication, LU-decomposition, FFT, and convolution are also provided.

## 3.3 Just-in-Time/Dynamic Translation

As mentioned previously, applications use the PeakStream APIs to specify mathematical operations to be performed on arrays. The VM translates those operations dynamically into parallel programs, on a just-in-time basis. Use of arrays as the fundamental data type in the PeakStream Platform, coupled with dynamic translation of programs, has the effect of decoupling the application programming model from the programming model of the processor being used. The VM has detailed knowledge of the specific processor being used (GPU/CPU/xPU). It performs optimizations necessary to make the application’s array-based code perform well, providing results with correct accuracy.

Note that actual computation is decoupled from the application API calls to the VM. Computation is typically deferred until a computationally intense parallel kernel can be created or until the application needs to read the computed data. Best performance will be achieved by applications that perform lots of computation on array data between reads of the computed results.

## 3.4 PeakStream Platform Headers and Libraries

The PeakStream Platform supports use of the C and C++ languages for application development. Language bindings to platform operations are provided by a set of header files and shared libraries.

The goal of the PeakStream APIs is to enable developers the ability to express parallel computations in a natural, easy-to-use manner. As such, there are some differences between the C and C++ APIs. The two sets of APIs, however, can interoperate and can be used at the same time by an application.

### 3.4.1 “peakstream.h” Header File

There is one header for application use: “peakstream.h.” It defines the PeakStream APIs for C and C++, specifically:

- The array datatypes and operations, as described previously.

- I/O operations, which move data into and out of PeakStream Arrays.

- Memory allocation interfaces, which allocate memory to be used for efficient I/O.

- Random number generation interfaces.

- Debugging and support interfaces.

- Error handling interfaces.

The PeakStream C++ bindings, along with brief examples of their use, are presented in section 3.5.

### 3.4.2 PeakStream Platform Libraries

A PeakStream application links against the PeakStream VM libraries. The libraries are dynamically linked with a stable Application Binary Interface. This means that the application does not need to be recompiled or relinked when a new version of the PeakStream VM is released.

Taken together, the use of array data types for computation, the use of dynamic translation, and the use of a stable dynamically linked library ABI, present a compelling proposition for the application developer. Specifically, an application developer can port code once to the PeakStream Platform and use it on whatever processor the platform supports, including future generations of processors that did not exist when the application was written.

## 3.5 C++ Interfaces

The PeakStream C++ API is tailored to support common C++ idioms. Arrays are represented by C++ objects, and operator overloading is used extensively to support a style of programming familiar to C++ developers. Array memory is managed via object creation and destruction and leads to a natural programming style.

### 3.5.1 C++ Interface Example

The following example shows a simple C++ code snippet that uses the PeakStream APIs to calculate the dot product of two vectors and return the resulting value. Dot product is calculated by multiplying the pairs of elements at the same position in the two input arrays, then summing the result of those multiplications. The result is a single scalar value.

#include <peakstream.h>

using namespace SP;

Arrayf32 dot_product_cxx(const Arrayf32& a,

const Arrayf32& b)

{

return sum(a * b, SP_ALL_DIMS);

}

In this example, the multiplication “a * b” is performed using an overloaded operator* that operates on PeakStream Array types. Note that the result is returned as a PeakStream Array that contains a scalar value, which ultimately enables the result to be used efficiently by subsequent computations on the processor.

Also note that in this example, a temporary array is allocated and de-allocated. The result of the multiplication “a * b” is allocated and de-allocated automatically on behalf of the application by the C++ compiler.

## 3.6 PeakStream Platform Tools

The PeakStream Platform provides debugging and profiling capabilities for application developers. It includes an execution Profiler and profile analyzer which can show where application cycles are being spent. Debugger interfaces are also provided and facilitate examination of PeakStream Arrays as they are being created and computed by the application. On Linux, these tools are provided in the form of extensions to gdb and a gprof style Profiler. On Microsoft® Windows®, these tools are provided as plug-ins to Microsoft Visual Studio®.

### 3.6.1 Profiling Tools

The PeakStream virtual machine (VM) can be configured to generate execution profiles while a PeakStream application is running. The execution profiles record the application’s I/O to and from the GPU, GPU time-consumed by the application or idle, CPU time used to generate parallel programs, and so on. Resource consumption is attributed back to application source lines and to PeakStream API calls within each application source line.

An offline profile analysis tool can be used to generate profiling reports from the saved profile data. The reports include application source code call points and the PeakStream API functions that are invoked, as well as per-function and cumulative resource consumption information. Most importantly, the Profiler reports include data that can be used to identify two important GPU performance bottlenecks: excessive I/O and parallel kernels that do not include enough computation to make full use of the processor. The Profiler also provides information on compute kernel synthesis, allowing the programmer to optimize compute kernel density.

### 3.6.2 Debugging Interfaces

The PeakStream Platform includes several debugging interfaces that can be used to examine PeakStream Array data while debugging an application program. These interfaces are provided as scripts and functions and can be invoked from the debuggers supported by the PeakStream Platform.

Although debugger support for examining array data may seem straightforward, several issues are involved that make doing so more complex when an accelerator such as a GPU is involved.

First and foremost, the PeakStream Array data may not have been computed immediately when requested. The PeakStream Platform treats the GPU as an asynchronous device, so there is no guarantee that the computation to generate the data will have completed when the debugger needs the data. In addition, the VM buffers computation to produce more computationally intense, parallel kernels so computation of the result might not even have been started when the result is requested. The VM must therefore ensure that the data is available when the debugger requests it.

Second, even if the array data has been computed, it may only exist in GPU memory. That memory is far away from the debugger that is actually debugging the application—typically across an I/O bus—that the debugger has no built-in facility to access. The VM must move data as necessary so that it is accessible to the debugger.

In addition to those two problems, the VM must take care not to perturb normal system operation to supply values to the debugger. The debugger may request data from an array that would not even be computed during normal operation. For instance, computation of temporary values will normally be folded into other operations being performed on the GPU and the temporary values will never be output. In that case, the VM must be able to provide the value that would be computed, while not disturbing the normal computations requested by the application.

# 4 Solutions to GPU Issues

As discussed earlier, application developers who want to harness the computational power of GPUs as accelerators face many challenges without a suitable application platform.. This section describes how the PeakStream Platform overcomes those technical challenges.

## 4.1 Software Stability when Processor Architecture Changes

First of all, because of its architecture, the PeakStream Platform insulates application developers from the architectural complexity of GPUs. Application developers never interact with the GPU directly and instead use the APIs that are implemented by the PeakStream VM.

The PeakStream VM transforms application API calls so that they generate optimized, target-specific code for whatever processor is in use. This transformation can take into account optimal parallel kernel size and can make use of optimized library routines.

Because PeakStream guarantees a stable application binary interface (ABI) across multiple platform versions, applications can move to new versions of the PeakStream Platform without recompiling. Applications can ultimately use any processor hardware supported by the latest version of the VM without recompilation.

Some overhead is implicit in the use of dynamic code generation. To keep this from adversely affecting application performance, the VM employs extensive caching so that commonly used code sequences can execute with very low overhead.

## 4.2 Tool Support and Compatibility

The PeakStream Platform provides important tools needed by application developers and verifies compatibility with third-party development tools.

As discussed earlier, the PeakStream Platform includes debugging and profiling tools that can analyze the I/O and compute hot-spots related to an application’s use of the GPU. These two tools are the primary ones that an application developer needs to achieve high performance from any multi-core processor.

In addition, the PeakStream Platform has been designed to work with third-party tools and libraries. Industry-standard compilers, including gcc, Microsoft Visual Studio and the Intel Compiler, are supported, as are the corresponding debuggers. Interoperability with communication libraries and other math libraries, such as common MPI libraries and the Intel Math Kernel Library (MKL), is also a feature of the PeakStream Platform. The interoperability allows developers to maintain their existing communication system and do piece-wise conversion of their applications to use the PeakStream APIs for computation. Finally, interoperability with software development and analysis tools such as CCov, gcov, and VTune [Intel03] has been designed into the PeakStream Platform so that application developers can find the hot spots in their code once compute is no longer their main cycle sink.

## 4.3 Accurate Mathematical Library Support

As discussed in Section 2.2.3, native GPU math libraries may not support the accuracy necessary for use in HPC applications. Part of the work in porting the PeakStream VM to each new multi-core target is examining the native math library and creating optimized, accurate replacement math library routines as needed.

Figure 2 shows the accuracy of a typical GPU native math library function, along with the accuracy of the corresponding version created for the PeakStream VM. Many GPU math library functions, including the transcendental and trigonometric functions, have similar accuracy issues. All are similarly improved for use in the PeakStream VM. The ability of the PeakStream Platform to provide accurate functions that work “out of the box” is of great benefit to the application developer.

## 4.4 Computationally Intense GPU Kernels

To be effective, GPU kernels must have substantially greater computational intensity than traditional serial CPU code. To achieve this level of computational intensity when coding directly for a particular multi-core target, an application developer would have to analyze the application code carefully and divide it into the right number of computational kernels and the optimization would have to stay within per-kernel limits on resource consumption and instruction count. These limits vary between different processors such as the ATI R580 and the ATI R600. While this is possible, it is very time consuming, especially in the absence of an ecosystem of supported performance analysis tools. Further, it does not port well from one processor to another.

The PeakStream VM creates computationally intense parallel kernels automatically for the application developer. Application developers write simple, mathematical functions as described in earlier and forthcoming examples. When the application runs, the VM looks at the set of operations performed by the application. The PeakStream VM automatically fuses operations together into computationally intense parallel kernels. Because the VM does this dynamically, based on the processor in use, it can create optimal kernels for whatever processor is available [Chan05][Riffel04].

## 4.5 Progressive Evaluation

The PeakStream VM hides the latency inherent in the loosely coupled GPU system architecture by allowing the application to continue while computation is being performed by the GPU. As discussed in Section 3.3, compilation and execution of parallel kernels are decoupled from application API calls. In other words, the application may make an API call to add two arrays that will be executed sometime in the future, just in time to return the needed result data back to the application. We call this feature of the platform “progressive evaluation.”

Progressive evaluation allows latency hiding. The application can issue a large number of compute requests into the VM and can continue processing other application work while the VM is performing the computations. That application work may include I/O (such as talking to other nodes in a compute cluster) or may include the generation of more compute work for the VM to process later.

To exploit the multiple CPUs common in today’s HPC systems, the VM employs multiple threads to handle compilation, execution, and data movement. These threads allow the application to continue its work—while in parallel—and the VM handles compilation of programs and asynchronous communication with the stream processor.

## 4.6 I/O Cost Analysis

Not all costs related to I/O with a GPU can be eliminated. But to achieve best performance, movement of data between the CPU and the GPU must be kept to a minimum. The PeakStream Platform encourages careful management of data movement in two ways: explicit interfaces and I/O analysis tools.

The PeakStream APIs include explicit data movement interfaces that move data between application arrays and PeakStream Arrays. The write operation copies application data into a PeakStream Array where it may be used for computation on an accelerator GPU. Similarly, the read operation reads data out of a PeakStream Array into an application array. Both read and write are capable of performing common scatter/gather operations to allow application flexibility in the way that they manage their data. Use of explicit APIs for I/O encourages the application developer to be cognizant of the I/O being done by the application.

As mentioned in Section 3.6.1, the PeakStream Profiler includes the ability to analyze application I/O to the GPU. By using the Profiler, application developers can very quickly identify GPU I/O in the program, but more importantly, they can prioritize it so that performance issues due to excessive I/O can be quickly resolved.

# 5 Examples of Using the PeakStream APIs

A primary advantage of using the PeakStream Platform is that it enhances developer productivity and reduces time-to-solution for solving computationally intensive problems such as Monte Carlo simulations of fixed income derivatives on Wall Street or seismic migration within the energy sector. There is a growing realization in the HPC development community that improving developer productivity is increasingly important [Kepner04]. This section walks through a sample application written in a serial fashion compared to an implementation using the PeakStream Platform.

## 5.1 Monte Carlo Options Pricing Source Code

Monte Carlo simulation is often used in financial markets to calculate the price of the stock option. The following source code sample shows an example of a traditional serialized code using the Intel MKL library [Intel04].

float sum1      = 0.0;

float deltat    = T/N;

float muDeltat  =

(rate-div-0.5*vol*vol)*deltat;

float volSqrtDeltat = vol*sqrt(deltat);

VSLStreamStatePtr stream;

float *deviate = new float[M];

float *tmp     = new float[M];

vslNewStream(&stream, VSL_BRNG_MRG32K3A,

1);

for(int j=0; j<M; j++)

tmp[j] = 0.0f;

for(int i=0; i<N; i++) {

vsRngGaussian(

VSL_METHOD_SGAUSSIAN_BOXMULLER2,

stream, M, deviate, 0.0f, 1.0f );

for(int j=0; j<M; j++) {

tmp[j] += deviate[j];

}

}

sum1 = 0.0;

float factr = log(price) + N * muDeltat;

for (j = 0; j<M; j++ ) {

float lnS1 = factr + volSqrtDeltat

* (tmp[j]);

float lnS2 = factr + volSqrtDeltat

* (-tmp[j]);

float S1 = exp(lnS1);

float S2 = exp(lnS2);

sum1 += 0.5*(max(0,S1-strike)

+ max(0,S2-strike));

}

return (exp(-rate*T)*sum1/M);

## 5.2 Monte Carlo Options Pricing Using Stream Programming

The following source code sample shows the same algorithm using the PeakStream stream programming model. Note that the PeakStream version is simple to read and understand.

float deltat   = T/N;

float muDeltat =

(rate-div-0.5*vol*vol)*deltat;

float volSqrtDeltat = vol*sqrt(deltat);

Arrayf32 meanSP; // result

{

RNGf32 rng_hndl(SP_RNG_CEICG12M6, 0);

Arrayf32 U = Arrayf32::zeros( M );

for(int i=0; i<N; i ++) {

U += rng_normal_make(rng_hndl, M);

}

Arrayf32 values;

{

float factr = log(price) + N * muDeltat;

Arrayf32 lnS1 = factr + volSqrtDeltat*U;

Arrayf32 lnS2 = factr + volSqrtDeltat*(-U);

Arrayf32 S1 = exp(lnS1);

Arrayf32 S2 = exp(lnS2);

values = (0.5 * (max( 0,S1-strike )

+ max( 0, S2-strike) ) * exp( -rate*T ));

}

meanSP = mean( values );

}

return(meanSP.read_scalar());

# 6 Related Work

There is no standard, universally accepted programming approach for multi-core processors. On CPUs, OpenMP is often used as a parallel approach, but it is recognized as being an overly complex programming model that is limited to thread-parallel processors. In addition:

- GPU programming approaches have been application-specific, targeting graphics, multimedia, or signal processing.

- GPU programming languages also have been architecture specific. The architectures of GPUs are very diverse, both in the types of the parallelism they support and in their memory hierarchies. Each typically supports multiple forms of parallelism, including traditional SIMD, four-way SIMD instructions large-scale and small-scale multi-threading, superscalar, and VLIW. Memory systems of GPUs offer different combinations and configurations of direct access to shared memory, direct memory access (DMA) access to shared memory, local memories, caches, and long and short vector registers.

# 6.1 Commercial GPU Shader Languages

Graphics developers program modern GPUs with either OpenGL [OpenGL05] or Microsoft Direct3D [DX9SDK] APIs, using software drivers provided by the GPU vendors. OpenGL and Direct3D refer to GPU programs as shaders. These APIs are very similar and provide two fundamental pieces of functionality: an API for compiling and executing shader programs and an API for traditional graphics functionality, such as rendering triangles, points, and quadrilaterals, loading texture maps, and so on. Shader programs are executed on each pixel as triangle rendering generates them for subsequent display.

Shader programs may either be written in C-like languages, such as Cg [Mark03,Fernando03], CUDA, HLSL [Peeper03], and GLSL [Rost04] or may be programmed with cross-platform pseudo-assembly languages, such as OpenGL ARB_Fragment_Program and Microsoft Direct3D® Shader Model Assembly. All of these shader languages share a number of common features:

- They provide an explicitly data-parallel programming model, not unlike MPL [Maspar91], where the developer manages parallelism and communication manually. The manual management of parallelism provides challenges for application developers.

- They are low-level languages that directly expose the features and limitations of the underlying hardware. They do not effectively abstract hardware resources, so when a program would exceed any architectural limitations, such as the number of available registers or the maximum program size, it fails to compile. As such, code written for one GPU under these languages requires substantial rewriting to get all the benefits of another GPU.

## 6.2 GPU Software Research Efforts

Over the years, the research community has produced a number of interesting software projects targeting GPUs, some of which share common features with the PeakStream Platform.

None of these projects is widely used in industry, however, because they are not supported (or, in some cases, not available) or are not of commercial quality in terms of features, robustness, and interoperability. They do not provide error-handling features, debugger and profiling support, numerically accurate implementations of many math functions, nor libraries of parallel numerical algorithms (such as matrix multiplication). Because the device interfaces and native instruction-set architectures of GPUs are not generally available, they all use the aforementioned commercial GPU APIs and shading languages internally to execute on GPUs.

### 6.2.1 Brook

The Brook language from Stanford University is a highly restrictive, implicitly parallel C-like language that makes streams and element-wise computational kernels explicit. Brook was intended for multimedia processing and high-performance computing [Buck04]. Unfortunately, due to its restrictive programming paradigm, some communication-intensive algorithms that could achieve high performance on today’s GPUs, such as logical unit (LU) decomposition, are difficult to express by using Brook. Moreover, new languages rarely gain widespread acceptance because they force developers to learn the new paradigm and syntax and to use new compilers and related tools, which most developers consider undesirable.

# 7 PeakStream: The New Generation of Multi-Core Systems

This white paper has discussed the PeakStream Platform, a new software platform that provides an easy-to-adopt stream programming model for the new generation of multi-core systems and provides easy access to accelerators such as GPUs. The PeakStream Platform implementation addresses many of the shortcomings of nontraditional co-processors such as the GPU through advanced portable VM technology and optimization techniques. The paper has shown how the PeakStream Platform provides well-designed APIs, portability between processors, a commercial-quality mathematical library, and useful development tools.

# References

[Allen06] ALLEN, E. et al., 2006. The Fortress Language Specification, version 0.866, Sun Microsystems, Inc.
http://research.sun.com/projects/plrg/fortress0866.pdf

[Akeley93] AKELEY, K., 1993. "Reality Engine Graphics," Proceedings of the 20th annual conference on computer graphics and interactive techniques, ACM, pp. 109-116.

[Buck04] BUCK, I. et al., 2004. "Brook for GPUs: stream computing on graphics hardware," Proceedings of SIGGRAPH 2004, Computer Graphics Proceedings, Annual Conference Series, ACM, 23, 3, 777-786.

[Chamberlain04] CHAMBERLAIN, B., 2004. "Chapel: The Cascade High Productivity Language," Language VMs ’04, workshop paper and presentation, October.

[Chan02] CHAN, E. et al., 2002. "Efficient partitioning of fragment shaders for multipass rendering on programmable graphics hardware," Proceedings of the ACM SIGGRAPH/EUROGRAPHICS conference on Graphics hardware, Eurographics Association.

[Charles05] CHARLES, P. et al., 2005. X10: "An Object-Oriented Approach to Non-Uniform Cluster Computing," Proceedings of the 20th Annual ACM SIGPLAN conference on Object-oriented programming, systems, languages, and applications, Annual Conference Series, ACM, 519-538.

[DP05] Data-Parallelism, 2005. Technical Report MSR-TR-2005-184, Microsoft Research, December.

[DX9SDK] DirectX 9 SDK, Microsoft Corp.

[Domine05] DOMINE, S., 2005. OpenGL Performance Tools.
http://download.nvidia.com/developer/presentations/2005/GDC/OpenGL_Day/OpenGL_Performance_Tools.pdf

[Eichenberger05] EICHENBERGER, A. et al., 2005. “Optimizing Compiler for the Cell Processor”, PACT 2005.

[Fernando03] FERNANDO, R. AND KILGARD, M.J., 2003. The Cg Tutorial: The Definitive Guide to Programmable Real-Time Graphics, Addison-Wesley.

[Flachs05] FLACHS, B. et al., Feb. 2005. "A Streaming Processing Unit for a CELL Processor," International Solid-State Circuits Conference Technical Digest .

[FSF06] FREE SOFTWARE FOUNDATION, 2006. GDB: The GNU Project Debugger. 
http://www.gnu.org/software/gdb/documentation

[FSF98] FREE SOFTWARE FOUNDATION, 1998. GNU gprof. 
http://www.gnu.org/software/binutils/manual/gprof-2.9.1/gprof.html

[Gordon02] GORDON M., et al., 2002. A Stream Compiler for Communication-Exposed Architectures, MIT/LCS Technical Memo LCS-TM-627.

[Govindarajum06] GOVINDARAJUM, N.K. et al., 2006. GPUTeraSort: High Performance Graphics Coprocessor Sorting for Large Database Management, Microsoft Research Technical Report (MSR-TR-2005-183), Accepted for publication in the Proceedings of the ACM SIGMOD International Conference on Management of Data. 
http://gamma.cs.unc.edu/GPUTERASORT/gputerasort_sigmod06.pdf

[Hanrahan06] HANRAHAN, P., 2006. "Why is graphics hardware so fast?," PPoPP ‘05: Proceedings of the tenth ACM SIGPLAN symposium on Principles and practice of parallel programming.

[Hennessy06] HENNESSY J. L., PATTERSON D. A., 2006. Compute Architecture: A Quantitative Approach, Morgan Kauffman.

[HPFF93] HIGH PERFORMANCE FORTRAN FORUM (HPFF), (July) 1993. High Performance Fortran Language Specification, Scientific Programming 2, 1, 1-170.

[Hillis89] HILLIS, W.D., 1989. The Connection Machine, The MIT Press, 
reprint edition.

[Hillis87] HILLIS, W.D. AND STEELE, J. G. L., 1987. "Data parallel algorithms," Communications of the ACM 30, 1, 78.

[Horn05] HORN, D.R. et al., 2005. "ClawHMMER: A Streaming HMMer-Search Implementation," Proceedings of the ACM/IEEE SC2005 Conference on Supercomputing. 
http://graphics.stanford.edu/papers/clawhmmer/hmmer.pdf

[Intel03] INTEL CORPORATION, 2003. Technologies for Measuring Software Performance (White paper 253445-001). 
http://cache-www.intel.com/cd/00/00/21/93/219307_measuring_performance.pdf

[Intel04] INTEL CORPORATION, 2004. Intel Math Kernel Library 9.0 
http://www.intel.com/software/products/mkl

[Kepner04] KEPNER, J., AND KOESTNER, D., 2004. HPCS Application Analysis and Assessment.

[Kolb05] KOLB, C. AND PHARR, M., 2005. Options pricing on the GPU, GPU Gems 2: Programming techniques for high-performance graphics and general-purpose computation, Fernando, R. and Pharr, M. eds., Addison Wesley Professional.

[Labonte04] LABONTE, F. et al. 2004. "The Stream Virtual Machine," Proceedings of 2004 International Conference on Parallel Architectures and Compilation Technique, PACT ’04, 267-277.

[Litvinov04] LITVINOV, V., 2004. "Streaming Virtual Machine and Two-Level Compilation Model for Streaming Architectures and Languages," Language VMs ’04 workshop paper and presentation, October.

[Mark03] MARK, W. et al., 2003. Cg: "A system for programming graphics hardware in a C-like language," Proceedings of SIGGRAPH 2003, Computer Graphics Proceedings, Annual Conference Series, ACM, 22, 3, 896-907.

[Maspar91] MASPAR COMPUTER CORPORATION, 1991. MasPar Fortran Reference Manual.

[McCool04] MCCOOL, M. AND DU TOIT, S., 2004. Metaprogramming GPUs with Sh, AK Peters, Ltd.

[McCool02] MCCOOL, M., ZHENG Q., AND POPA T.S., 2002. "Shader Metaprogramming," SIGGRAPH/Eurographics Graphics Hardware Workshop, Saarbruecken, Germany, September 2-3, 57-68.

[OpenGL05] OpenGL Architecture Review Board, 2005. OpenGL Programming Guide: The Official Guide to Learning OpenGL, Version 2 (5th Edition), Addison-Wesley Professional.

[Peeper03] PEEPER, C. AND MITCHELL, J.L., 2003. Introduction to the DirectX 9 high level shading language, ShaderX2 - Introduction and Tutorials with DirectX 9.0, Engel, W. F. ed., Wordware, September.

[Peercy00] PEERCY, M.S. et al., 2000. "Interactive multi-pass programmable shading," Proceedings of SIGGRAPH 2000, Computer Graphics Proceedings, Annual Conference Series, ACM, 19, 3, 425-432.

[Pham05] PHAM, D. et al., 2005 (Feb). "The Design and Implementation of a 
First-Generation CELL Processor," International Solid-State Circuits Conference Technical Digest.

[Quinlan00] QUINLAN, D., 2000. "ROSE: compiler support for object-oriented frameworks," Parallel Processing Letters, 10, (2,3), 215-226.

[Reynders96] REYNDERS, J. et. al., 1996. Pooma: A framework for scientific simulation on parallel architectures, Parallel Programming using C++, Wilson, G., and Lu, P., Eds., M.I.T. Press, 553-594.

[Riffel04] RIFFEL, A. et al., 2004. "Mio: fast multipass partitioning via priority-based instruction scheduling," Proceedings of the ACM SIGGRAPH/EUROGRAPHICS conference on Graphics hardware, ACM Press.

[Rose87] ROSE, J. R., et al., May 1987. C*: "An Extended C Language for Data Parallel Programming," Proceedings of the Second International Conference on Supercomputing, L. P. Kartashev et al., Eds., 2–16.

[Rost04] ROST, R., 2004. OpenGL Shading Language, Addison-Wesley Professional.

[Sankaralingam03] SANKARALINGAM, K. et al., 2003. “Exploiting ILP, TLP, and DLP Using Polymorphism in the TRIPS Architecture”, 30th Annual International Symposium on Computer Architecture (ISCA), pp. 422-433.

[Sarkar04] SARKAR, V., 2004. X10: A Programming Model for Hierarchical Heterogeneous Parallelism, Language VMs ’04 workshop paper and presentation, October.

[Scheidegger04] SCHEIDEGGER, C. et al., 2004. "Navier-Stokes on Programmable Graphics Hardware Using SMAC," Proceedings of XVII SIBGRAPI - II SIACG 2004, 300-307.
http://www.inf.ufrgs.br/~comba/papers/2004/smac.pdf

[Smith06] SMITH, A. et al., 2006. “Compiling for EDGE Architectures”, 2006 International Conference on Code Generation and Optimization (CGO).

[Tarditi05] TARDITI, D., et al., 2005. Accelerator: Simplified Programming of Graphics Processing Units for General-Purpose Uses Via Data-Parallelism. 
http://research.microsoft.com/research/pubs/view.aspx?type=Technical%20Report&id=1040

[Taylor02] Taylor M. et al., 2002. “The Raw Microprocessor: A Computational Fabric for Software Circuits and General Purpose Programs”, IEEE Micro, Vol 22, Issue 2.

[TMC86] THINKING MACHINES CORPORATION, 1986. C* Programming Manual.

[Veldhuizen98] VELDHUIZEN, T.L. AND GANNON, D., 1998. "Active libraries: Rethinking the roles of compilers and libraries," Proceedings of the SIAM Workshop on Object Oriented Methods for Inter-operable Scientific and Engineering Computing (OO’98), SIAM Press.

[Ye04] YE, Z. et al., 2004. Lattice-Based Flow Simulation on GPU.
http://www.cs.sunysb.edu/~vislab/projects/gpgpu/GPULBM.pdf