@PARAFILTR ON =

@Z_TOC TITLE = Abstract

This report describes several changes and additions proposed for the Network Time Protocol Version 4. Previous versions of this protocol have been widely deployed in the Internet and a considerable body of experience has accumulated in their use. The proposed new version contains several improvements to existing algorithms, together with new features designed to support multicast modes of operation, fully distributed peer subnets and automated NTP peer discovery and subnet configuration.

This report describes work in progress and is produced in order to stimulate discussion and refinement of the various algorithms. Some of the algorithms, in particular the work on refining the NTP local clock model, has already been implemented and now in test. Other work, including multicast support, is in the prototype phase, but needs further development and consensus on key management and distribution. Other work is still in the formative stage, including the fully distributed modes of operation and peer discovery and subnet configuration.

It is anticipated that further development in these areas will continue while discussion and consensus is reached on the final form of the protocol and its algorithms. When complete, the final design will be submitted for approval by the Internet standards apparatus.

Keywords: computer clock, time synchronization, network synchronization.

Sponsored by: Advanced Research Projects Agency under NASA Ames Research Center contract NAG 2-638, National Science Foundation grant NCR-93-01002 and Northeastern Center for Engineering Education contract A30327-93.

@Z_TOC TITLE = Table of Contents

@Z_TOC LVL 2 = 1.	 Introduction	1<$!~3>

@Z_TOC LVL 2 = 2.	 NTP Local Clock Discipline Algorithm	1<$!~3>

@Z_TOC LVL 3 = 2.1.	 Typical Oscillator Characteristics	1<$!~3>

@Z_TOC LVL 3 = 2.2.	 Controlling Time and Frequency	3<$!~5>

@Z_TOC LVL 3 = 2.3.	 Avoiding Old Data	6<$!~8>

@Z_TOC LVL 3 = 2.4.	 Increasing the Frequency Tolerance	6<$!~8>

@Z_TOC LVL 3 = 2.5.	 Improving Stability at Initial Startup	7<$!~9>

@Z_TOC LVL 3 = 2.6.	 Specification of the Local Clock Discipline Algorithm	8<$!~10>

@Z_TOC LVL 3 = 2.7.	 Determining the Poll Interval	10<$!~12>

@Z_TOC LVL 2 = 3.	 Mitigation Rules	11<$!~13>

@Z_TOC LVL 3 = 3.1.	 The Prefer Peer	12<$!~14>

@Z_TOC LVL 3 = 3.2.	 The Pulse-per-Second (PPS) Signal	13<$!~15>

@Z_TOC LVL 3 = 3.3.	 The PPS Kernel Discipline	14<$!~16>

@Z_TOC LVL 2 = 4.	 New Radio Clock Drivers	15<$!~17>

@Z_TOC LVL 3 = 4.1.	 NIST Automated Computer Time Service	17<$!~19>

@Z_TOC LVL 3 = 4.2.	 PPS Clock Discipline	18<$!~20>

@Z_TOC LVL 2 = 5.	 Alternate Modes for NTP	19<$!~21>

@Z_TOC LVL 3 = 5.1.	 Reasons for Multicasting Time	20<$!~22>

@Z_TOC LVL 3 = 5.2.	 NTP Protocol Refinements:	21<$!~23>

@Z_TOC LVL 3 = 5.3.	 Protocol State Machine	21<$!~23>

@Z_TOC LVL 3 = 5.4.	 Hierarchical Subnets	24<$!~26>

@Z_TOC LVL 3 = 5.5.	 Multicast Group Address Assignment and Group Address Reuse	25<$!~27>

@Z_TOC LVL 3 = 5.6.	 Authentication	25<$!~27>

@Z_TOC LVL 4 = 5.6.1.	 Authentication in Client-Server Mode	26<$!~28>

@Z_TOC LVL 4 = 5.6.2.	 Authentication in Broadcast Mode	27<$!~29>

@Z_TOC LVL 3 = 5.7.	 Strawman Model	28<$!~30>

@Z_TOC LVL 2 = 6.	 Future Improvements	28<$!~30>

@Z_TOC LVL 3 = 6.1.	 NTP Distributed Mode	29<$!~31>

@Z_TOC LVL 2 = 7.	 References	31<$!~33>

@Z_TOC LVL 1 = Appendix A. NTP Simulator Source Program Listing	33<$!~35>

@Z_TOC TITLE = List of Figures

@Z_TOC LVL 5 = Figure 1. Allan Variance of Typical Local Oscillator	2<$!~4>

@Z_TOC LVL 5 = Figure 2. Measuring Delay and Offset	27<$!~29>