@PARAFILTR ON =

@Z_TOC TITLE = Abstract

The Highball Project, sponsored by ARPA/CSTO, has been investigating architectures suitable for networks spanning large geographic areas and capable of very high data rates. The Highball network architecture is based on a high speed crossbar switch and an adaptive, distributed, TDMA scheduling algorithm. The scheduling algorithm controls the instantaneous configuration and dwell time of the switch, one of which is attached to each node. In order to send a single burst or a multi-burst packet, a reservation request is sent to all nodes. The scheduling algorithm then configures the switches immediately prior to the arrival of each burst, so it can be relayed immediately without requiring local storage. Reservations and housekeeping information are sent using a special broadcast-spanning-tree schedule.

Progress to date in the Highball Project includes the design and testing of a suite of scheduling algorithms, construction of software reservation/scheduling simulators, and construction of a strawman hardware and software implementation. A prototype switch controller and timestamp generator have been completed and are in test. Detailed documentation on the algorithms, protocols and experiments conducted are given in various reports and papers published. Abstracts of this literature are included in the bibliography at the end of this report, which serves as an extended executive summary.

Keywords: supercomputer networks, gigabit networks, reservation-TDMA, Highball architecture.

Sponsored by: Advanced Research Projects Agency under NASA Ames Research Center contract NAG 2-638, National Science Foundation grant NCR-93-01002 and Northeastern Center for Engineering Education contract A30327-93.

@PAGE =

@Z_TOC TITLE = Table of Contents

@Z_TOC LVL 2 = 1.	 Introduction and Background	1<$!~4>

@Z_TOC LVL 2 = 2.	 Design Approach	2<$!~5>

@Z_TOC LVL 2 = 3.	 Network Architecture	3<$!~6>

@Z_TOC LVL 2 = 4.	 Project Status and Results	4<$!~7>

@Z_TOC LVL 3 = 4.1.	 Highball Scheduling Issues	4<$!~7>

@Z_TOC LVL 3 = 4.2.	 Comparison With Other Scheduling Algorithms	5<$!~8>

@Z_TOC LVL 3 = 4.3.	 Control Channel Design	6<$!~9>

@Z_TOC LVL 3 = 4.4.	 Error Modeling and Robustness Issues	7<$!~10>

@Z_TOC LVL 3 = 4.5.	 Highball Time Synchronization	8<$!~11>

@Z_TOC LVL 4 = 4.5.1.	 Precision Time Calibration and Distribution	10<$!~13>

@Z_TOC LVL 4 = 4.5.2.	 Kernel Modifications for Improved Performance	11<$!~14>

@Z_TOC LVL 5 = 4.5.2.1.	 Improved Precision for Unix Kernels	11<$!~14>

@Z_TOC LVL 5 = 4.5.2.2.	 New Operating System Interface	12<$!~15>

@Z_TOC LVL 5 = 4.5.2.3.	 Software Timekeeping Improvements	12<$!~15>

@Z_TOC LVL 5 = 4.5.2.4.	 Interfaces for Special Signals	13<$!~16>

@Z_TOC LVL 4 = 4.5.3.	 Reducing Frequency Wander of the System Clock	13<$!~16>

@Z_TOC LVL 5 = 4.5.3.1.	 PPS Signal Discipline	14<$!~17>

@Z_TOC LVL 5 = 4.5.3.2.	 External Hardware Clock	14<$!~17>

@Z_TOC LVL 5 = 4.5.3.3.	 Future Improvements for NTP Version 4	15<$!~18>

@Z_TOC LVL 3 = 4.6.	 Hardware Development	15<$!~18>

@Z_TOC LVL 3 = 4.7.	 Software Development	17<$!~20>

@Z_TOC LVL 2 = 5.	 Future Work	18<$!~21>

@Z_TOC LVL 2 = 6.	 Technology Transfer	19<$!~22>

@Z_TOC LVL 2 = 7.	 Other Activities	19<$!~22>

@Z_TOC LVL 2 = 8.	 References and Bibligraphy Prior to 1990	21<$!~24>

@Z_TOC LVL 2 = 9.	 Bibliography of Publications Supported by this Contract	22<$!~25>

@Z_TOC LVL 3 = 9.1.	 Publications (with abstracts) in 1990	22<$!~25>

@Z_TOC LVL 3 = 9.2.	 Publications (with abstracts) in 1991	23<$!~26>

@Z_TOC LVL 3 = 9.3.	 Publications (with abstracts) in 1992	24<$!~27>

@Z_TOC LVL 3 = 9.4.	 Publications (with abstracts) in 1993	25<$!~28>

@Z_TOC LVL 3 = 9.5.	 Publications (with abstracts) in 1994	25<$!~28>

@Z_TOC LVL 3 = 9.6.	 Publications (with abstracts) in 1995	28<$!~31>