@PARAFILTR ON =

@Z_TOC TITLE = Abstract

This document describes a network architecture called Highball and a preliminary design for a prototype, wide-area data network designed to operate at speeds of 1 Gbps and beyond. It is intended for applications requiring high speed burst transmissions where some latency between requesting a transmission and granting the request can be anticipated and tolerated. Examples include real-time video and disk-disk transfers, national filestore access, remote sensing and similar applications. The network nodes include an intelligent crossbar switch, but have no buffering capabilities; thus, data must be queued at the end nodes. There are no restrictions on the network topology, link speeds or end-end protocols. The end systems, nodes and links can operate at any speed up to the limits imposed by the physical facilities.

This document presents an overview of an initial design approach and is intended as a benchmark upon which a detailed design can be developed. It describes the network architecture and proposed access protocols, as well as functional descriptions of the hardware and software components that could be used in a prototype implementation. It concludes with a discussion of additional issues to be resolve in continuing stages of this project.

Keywords: supercomputer networks, gigabit networks, reservation-TDMA, Highball architecture.

Sponsored by: Defense Advanced Research Projects Agency under NASA Ames Research Center contract number NAG 2-638 and National Science Foundation grant number NCR-89-13623.

@PAGE =

@Z_TOC TITLE = Table of Contents

@Z_TOC LVL 1 = 1.	 Introduction	1

@Z_TOC LVL 2 = 1.1.	 Overview	2

@Z_TOC LVL 2 = 1.2.	 Traffic Models	3

@Z_TOC LVL 2 = 1.3.	 Design Approach	4

@Z_TOC LVL 1 = 2.	 Network Architecture	6

@Z_TOC LVL 2 = 2.1.	 Network Protocols	8

@Z_TOC LVL 3 = 2.1.1.	 Slotted-ALOHA Mode	8

@Z_TOC LVL 3 = 2.1.2.	 Fixed-TDMA Mode	9

@Z_TOC LVL 3 = 2.1.3.	 R-TDMA Mode	10

@Z_TOC LVL 2 = 2.2.	 Scheduling Algorithms	11

@Z_TOC LVL 3 = 2.2.1.	 Breadth-First Algorithms	12

@Z_TOC LVL 3 = 2.2.2.	 Incremental Algorithms	12

@Z_TOC LVL 3 = 2.2.3.	 Other Algorithms	13

@Z_TOC LVL 2 = 2.3.	 Burst Formatting and Framing	13

@Z_TOC LVL 2 = 2.4.	 Scheduling Operations	15

@Z_TOC LVL 2 = 2.5.	 Node Synchronization	16

@Z_TOC LVL 1 = 3.	 Hardware Overview	17

@Z_TOC LVL 2 = 3.1.	 Crossbar Switch, Controller and Interfaces	17

@Z_TOC LVL 2 = 3.2.	 Data Transceiver and Controller	18

@Z_TOC LVL 2 = 3.3.	 Master Clock	21

@Z_TOC LVL 2 = 3.4.	 Node Processor	21

@Z_TOC LVL 2 = 3.5.	 Link Simulator	22

@Z_TOC LVL 1 = 4.	 Software Overview	23

@Z_TOC LVL 2 = 4.1.	 Normal Operation	24

@Z_TOC LVL 2 = 4.2.	 Link and Node Synchronization	24

@Z_TOC LVL 2 = 4.3.	 Network Synchronization	25

@Z_TOC LVL 2 = 4.4.	 Schedule Synchronization	26

@Z_TOC LVL 1 = 5.	 Scheduling Algorithms	27

@Z_TOC LVL 1 = 6.	 Simulation	28

@Z_TOC LVL 2 = 6.1.	 A Special-Purpose Simulator	29

@Z_TOC LVL 2 = 6.2.	 Preliminary Simulation Results	30

@Z_TOC LVL 1 = 7.	 Development Beyond the Initial Prototype Network	31

@Z_TOC LVL 2 = 7.1.	 Getting Fast	31

@Z_TOC LVL 2 = 7.2.	 Reducing Processing Requirements	32

@Z_TOC LVL 2 = 7.3.	 Reducing Reservation Delays	33

@Z_TOC LVL 1 = 8.	 References	33

@Z_TOC TITLE = List of Figures

@Z_TOC LVL 5 = Figure 1. NSFNET Backbone Network	4

@Z_TOC LVL 5 = Figure 2. Reservation TDMA Network	6

@Z_TOC LVL 5 = Figure 3. Burst Schedules	13

@Z_TOC LVL 5 = Figure 4. Burst Format	14

@Z_TOC LVL 5 = Figure 5. Node Controller	17

@Z_TOC LVL 5 = Figure 6. Switch Controller	18

@Z_TOC LVL 5 = Figure 7. Transceivers and Input/Output Controller	19

@Z_TOC LVL 5 = Figure 8. Master Clock	20

@Z_TOC LVL 5 = Figure 9. Node Processor	21

@Z_TOC LVL 5 = Figure 10. Link Simulator	22

@Z_TOC LVL 5 = Figure 11. Link Simulator Detail	22

@Z_TOC LVL 5 = Figure 12. Software Queues	23

@Z_TOC LVL 5 = Figure 13. Throughput-Delay	30

@Z_TOC TITLE = List of Tables

@Z_TOC LVL 5 = Table 1. Scheduling Times	28