@CHAPTER = Appendix <$R[C#]>. Program Listing

/*

***********************************************************************

*                                                                     *

* Program to control LORAN-C radio                                    *

*                                                                     *

* This program controls a special-purpose radio designed to receive   *

* transmissions from the US Coast Guard LORAN-C navigation system.    *

* These stations operate on an assigned radio frequency of 100 kHz    *

* and can be received over the continental US, adjacent coastal areas *

* and significant areas elsewhere in the world.                       *

*                                                                     *

* The radio, which is contained in a separate, shielded box, receives *

* the signals, which consist of an eight-pulse biphase-modulated      *

* pulse group transmitted at a 1-kHz rate. Each of these pulse groups *

* is repeated at an interval characteristic of the particular LORAN-C *

* chain, which consists of a master station and up to three slave     *

* stations. The radio includes a synchronous detector driven by a     *

* quadrature-phase clock, two integrators with adjustable gain and    *

* an signal-level detector used to derive the agc voltage.            *

*                                                                     *

* The radio is controlled by this program using a special-purpose     *

* interface, which converts the receiver signals using an             *

* analog/digital converter and multiplexor, and generates the digital *

* timing and analog control signals using an AMD 9513A System Timing  *

* Controller (STC) chip, two digital/analog converters and            *

* miscellaneous logic components. The radio provides three analog     *

* signals, one for the in-phase integrator, another for the           *

* quadrature-phase integrator and a third for the agc. This program   *

* computes the master oscillator frequency adjustment and receiver    *

* agc voltage.                                                        *

*                                                                     *

* The reciever includes a precision, oven-controlled crystal          *

* oscillator used to derive all timing signals used by the receiver   *

* and this program. The 5-MHz output of this oscillator is adjusted   *

* over a small range by this program to coincide with the LORAN-C     *

* signal as broadcast to within a few parts in 10e10 and is suitable  *

* for use as a laboratory frequency standard. The oscillator itself   *

* should have good intrinsic stability and setability to within less  *

* than 2.5 Hz at 5 MHz (0.5 ppm), since it must maintain the master   *

* clock to within 100 us over the pulse-code scan interval of several *

* minutes.                                                            *

*                                                                     *

* The PC running this program generates the control signals necessary *

* to run the radio and produces a 1-pps signal synchronized to        *

* UTC(LORAN) to within a fraction of a microsecond. When manually     *

* adjusted using time-of-coincidence (TOC) data published by US Naval *

* Observatory, this signal is suitable for use as a precision source  *

* of standard time. The system can generate all sorts of external     *

* signals as well, as programmed in the 9513A.                        *

*                                                                     *

*      David L. Mills (mills@udel.edu) 27 March 1992                  *

*                                                                     *

***********************************************************************

*

* Current LORAN-C chains by gri (master listed first)

*

* 9990 North Pacific		St. Paul Island, Attu, Port Clarence,

*							Narrow Cape

* 9980 Icelandic Sea

* 9970 North West Pacific	Iwo Jima, Marcus Island, Hokkaido, Gesashi,

*							Yap Island

* 9960 North East US		Seneca, Caribou, Nantucket, Carolina Beach,

*							Dana

* 9940 West Central US		Fallon, George, Middletown, Searchlight

* 9610 South Central US

* 8970 Great Lakes US		Dana, Malone, Seneca, Baudette

* 8290 North Central US

* 7990 Mediterranian Sea	Sellia Marina, Lampedusa, Kargabarun,

*							Estartit

* 7980 South East US		Malone, Grangeville, Raymondville, Jupiter,

*							Carolina Beach

* 7970 Norwegian Sea		Ejde, Sylt, B0, Sandur, Jan Mayen

* 7960 Gulf of Alaska		Tok, Narrow Cape, Shoal Cove

* 7930 Labrador Sea		Angissoq, Sandur, Ejde, Cape Race

* 5990 West Central Canada	Williams Lake, Shoal Cove, Port Hardy

* 5930 East Central Canada	Caribou, Nantucket, Cape Race

* 4990 Central Pacific		Johnson Island, Upolu Point, Kure Island

*/

#include <<stdio.h>>

#include <<ctype.h>>

#include <<bios.h>>

#include <<math.h>>

#include <<conio.h>>

#include <<string.h>>

/*

* Sizes of things. The pulse-group filter is a shift register with one

* stage for each 100-us bin in a 1000-us sample window plus two stages

* for a noise gate. The envelope filters consist of one stage for each

* 10-us cycle in the 300-us envelope pulse gate, for a total of 30

* samples.

*/

#define NRMS 10					/* size of pulse-group filter */

#define NENV 30					/* size of envelope filter */

/*

* Program characteristics. The field and display guard times are the

* maximum latency for the program to process samples at the end of a

* gri and for the output routines to print a line, respectively. If the

* time to the next gri is less than either of these, the next frame is

* skipped. The watchdog timeout is the maximum number of frames before

* the program abandons cycle search and reverts to pulse-group seach.

* The agc averaging factor sets the time constant of integration for

* the receiver signal- level indicator. The remaining parameters

* establish the minimum and maximum median filter size and envelope and

* phase weights.

*/

#define FGUARD 1000				/* field guard time (100 us) */

#define DGUARD 8000				/* display guard time (100 us) */

#define WATCHDOG 2000			/* watchdog timeout (frame) */

#define AGCFAC 16				/* agc averaging factor */

#define MMIN 3					/* min median filter size */

#define MMAX 10					/* max median filter size */

#define EMIN 5					/* min envelope weight */

#define EMAX 50					/* max envelope weight */

#define EFAC 1.1				/* envelope adjustment factor */

#define PMIN 5					/* min phase weight */

#define PMAX 200				/* max phase weight */

#define PFAC 1.2				/* phase adjustment factor */

/*

* Receiver characteristics. The receiver delay is characteristic of the

* receiver bandpass. The pulse-group offset is characteristic of the

* pulse-group filter and noise gate.

*/

#define RCVDELAY 50				/* receiver delay (200 ns) */

#define OFFSET 21				/* pulse-group offset (10 us) */

/*

* Receiver gain and noise gates. The vco parameter is adjusted

* for zero nominal frequency offset. The agc parameter is adjusted for

* nominal receiver output on the peak loran pulse of 800 mv p-p. The

* receiver gain parameter is adjusted so that the agc threshold (knee)

* occurs at a peak signal amplitude of 100, as determined by the status

* display. The derived envelope factor is adjusted so the zero crossing

* of the derived envelope signal occurs at the third cycle. The noise

* gate parameters establish the error/false-alarm rates.

*/

#define VCO 194					/* initial vco dac */

#define AGC 162					/* initial agc dac */

#define RGAIN 2.5				/* receiver gain */

#define DERVEL 2.2908			/* derived envelope factor */

#define PGATE 3					/* pulse-group noise gate */

#define SGATE 2					/* strobe noise gate */

/*

* The receiver agc is controlled to produce a q-channel amplitude of

* 100, which represents a demodulator transfer function at the third

* carrier cycle of 50 V/rad. The vco has a sensitivity of 1 Hz/V

* reduced to 0.1 Hz/V by a pad between the dac and the vco, which

* represents a transfer function of 0.628 rad/V-s. The dac produces 6 V

* p-p for an input of 256V p-p, for a transfer function of .0234. The

* ratio of the 100-kHz demodulator clock to the vco frequency (5 MHz)

* is 1/50. The overall pll gain is the product of these factors .0147,

* rounded up to .015 for neatness.

*/

#define VGAIN .015				/* overall loop gain */

/*

* Timing generator definitions

*/

#define PORT 0x0300				/* controller port address */

#define TGC PORT+0				/* stc control port (r/w) */

#define TGD PORT+1				/* stc data port (r/w) */

/*

* Analog/digital converter definitions

*/

#define ADC PORT+2				/* adc buffer (r)/address (w) */

#define ADCGO PORT+3			/* adc status (r)/adc start (w) */

#define START 0x01			/* converter start bit (w) */

#define BUSY 0x01			/* converter busy bit (r) */

#define DONE 0x80			/* conversion done bit (r) */

/*

* Digital/analog converter definitions

* Note: output voltage is inverted from buffer

*/

#define DACA PORT+4				/* vco (dac a) buffer (w) */

#define DACB PORT+5				/* agc (dac b) buffer (w) */

/*

* Code generator definitions

* Note: bits are shifted out from the lsb first

*/

#define CODE PORT+6				/* pulse-code buffer (w) */

#define MPCA 0xCA			/* LORAN-C master pulse code group a */

#define MPCB 0x9F			/* LORAN-C master pulse code group b */

#define SPCA 0xF9			/* LORAN-C slave pulse code group a */

#define SPCB 0xAC			/* LORAN-C slave pulse code group b */

/*

* Mode register definitions

*/

#define PAR PORT+7				/* parameter buffer (w) */

#define INTEG 0x03			/* integrator mask */

/*

* time constant values

*

* 0	1.000 ms

* 1	0.264 ms

* 2	0.036 ms

* 3	short caps

*/

#define GATE 0x0C			/* gate source mask */

/*

* gate source values

*

* 4	always open

* 8	group repeition interval (GRI)

* c	puldse code interval (PCI)

* f	strobe (STB)

*/

#define IEN 0x20			/* enable interrupt bit */

#define EN5 0x40			/* enable counter 5 bit */

#define ENG 0x80			/* enable gri bit */

/*

* Timing generator (STC) commands

*/

/* argument sssss = counter numbers 5-1 */

#define LOADDP 0x00				/* load data pointer */

/* argument ee = element (all groups except ggg = 000 or 111) */

#define MODEREG 0x00		/* mode register */

#define LOADREG 0x08		/* load register */

#define HOLDREG 0x10		/* hold register */

#define HOLDINC 0x18		/* hold register (hold cycle increm) */

/* argument ee = element (group ggg = 111) */

#define ALARM1 0x07			/* alarm register 1 */

#define ALARM2 0x0F			/* alarm register 2 */

#define MASTER 0x17			/* master mode register */

#define STATUS 0x1F			/* status register */

#define ARM 0x20				/* arm counters */

#define LOAD 0x40				/* load counters */

#define LOADARM 0x60			/* load and arm counters */

#define DISSAVE 0x80			/* disarm and save counters */

#define SAVE 0xA0				/* save counters */

#define DISARM 0xC0				/* disarm counters */

/* argument nnn = counter number */

#define SETTOG 0xE8				/* set toggle output HIGH for counter */

#define CLRTOG 0xE0				/* set toggle output LOW for counter */

#define STEP 0xF0				/* step counter */

/* argument eeggg, where ee = element, ggg - counter group */

/* no arguments */

#define ENABDPS 0xE0			/* enable data pointer sequencing */

#define ENABFOUT 0xE6			/* enable fout */

#define ENAB8 0xE7				/* enable 8-bit data bus */

#define DSABDPS 0xE8			/* disable data pointer sequencing */

#define ENAB16 0xEF				/* enable 16-bit data bus */

#define DSABFOUT 0xEE			/* disable fout */

#define ENABPFW 0xF8			/* enable prefetch for write */

#define DSABPFW 0xF9			/* disable prefetch for write */

#define RESET 0xFF				/* master reset */

/*

* Function declarations

*/

void status(double, double, char*);

/*

* STC setup. Note gri = 99600, pci = 300 and stb = 10 (us).

*

* Counter 1 generates a 200-kHz signal from the 5-MHz master VCO. This

* signal is a slightly assymetrical square wave (duty factor 12/13). An

* external flipflop divides this signal by two to get the 100-kHz gri

* clock which drives counter 2. All other counters are driven from the

* 5-MHz source. The 200-kHz and 100-kHz signals are used by the

* synchronous demodulator in the receiver.

*

* Counter 2 generates the gri (pulse-code) gate, which repeats at the

* interval assigned to the LORAN-C chain; e.g., 9960 for the Northeast

* U.S. chain. The signal consists of a high 8-ms interval preceeded by

* a programmable low interval normally equal to the gri interval less 8

* ms. Counter 3 generates the pulse-code (pci) gate, which enables the

* receiver only when a pulse group is expected. The signal consists of

* eight 300-us high intervals beginning at the high interval of counter

* 2. Counter 4 generates the stb (cycle) gate used during envelope

* scan. The signal consists of a high 10-us interval preceeded by a

* programmable interval in the range up to about 300 us.

*

* Counter 5 operates as a gated divider to drive the frequency scalar

* and output divider, which produces the output signals for external

* equipment. The gating signal is generated by counter 4, which can be

* enabled for this purpose under probram control. When so enabled,

* counter 5 is stopped for the interval programmed in counter 4,

* enabling precise alignment of the frequency scalar and output divider

* to UTC. The output signal can be at 1 pps and any decimal multiple up

* to 100 kHz plus 5 MHz or, if UTC alignment is not necessary, any

* binary or decimal submultiple of 5 MHz. Note that all counters

* operate in binary mode, except the frequency scalar and output

* divider, which normally operate in bcd mode.

*/

int init[] = {

0x0162,   12,   13,			/* counter 1 (p0) */

0x0262, 9160,  800,			/* counter 2 (gri) */

0x8162, 1500, 3500,			/* counter 3 (pcx) */

0xc162,    0,   50,			/* counter 4 (stb) */

0x0162,   25,   25 			/* counter 5 (out) */

};

/*

* Standard envelope cycle amplitudes. These are matched with the

* received envelope amplitudes to compute the rms error, assuming the

* reference cycle (3) is at the reference phase in the envelope window.

* Following are the cycle amplitude values (peak-normalized to 100) in

* the standard LORAN transmission specification.

*

*         cycle     1     2     3*    4     5     6     7 */

double envcyc[7] = {4.7, 25.3, 50.8, 72.9, 88.6, 97.3,100.0};

/*

* Program variables (units)

*/

int ptrenv = 0;					/* index of display cycle */

char mode = '1';				/* operating mode */

char pulse = 'm';				/* master (m) or slave (s) codes */

int par = 0x0a;					/* mode register */

char kbd = ' ';					/* latest keystroke */

/*

* System timing and rrelated data (units). These values provide the

* precise offset of the reference phase relative to the epoch when

* the STC chip was last reset by this program. All timing calculations

* are performed relative to this epoch. The freq and phase variables

* are computed in various places in the program, but take effect only

* at the end of the processing cycle, so that all timing calculations

* can be performed with respect to the epoch the system is actually at.

*/

int frame = 0;					/* offset to reference frame (2*gri) */

int offset = 0;					/* offset to reference gri (10 us) */

int strobe = 0;					/* offset to reference cycle (10 us) */

int gri = 9960;					/* group repetition interval (10 us) */

int pcx = 1500;					/* pulse-code interval (200 ns) */

int freq = 0;					/* frequency offset (10 us/frame) */

int phase = 0;					/* phase offset (10 us) */

int step = 1;					/* phase step (10 us) */

int stb[MMAX];					/* strobe median filter */

int sgate = SGATE;				/* strobe noise (max-min) */

/*

* Various controls normally preset, but can be adjusted by keyoard

* commands. After all, this is a prototype device.

*/

double vco = VCO;				/* vco dac signal */

double vcodac = VCO;			/* vco dac bias (dac a) */

double agc = AGC;				/* agc dac signal */

double agcdac = AGC;			/* agc dac bias (dac b) */

/*

* Raw and processed data input from receiver. The raw data are received

* directly from the adc and summed for both the a and b gri intervals.

* The offset data are computed during the receiver calibration modes

* and used to remove bias from the raw data to produce the net signal

* valid at the end of the frame. As the envelope pointer cycles through

* all 30 100-us cycles of the pulse gate, the i and q signal envelopes

* are averaged separately for use in the cycle-identification and

* phase-tracking processes. The median filters are used to suppress

* impulse-noise and pulse-dropout. The rms error signal produced from

* the i and q signals is used during the pulse-code identification

* process.

*/

double isig = 0;				/* i-signal (a+b) */

double qsig = 0;				/* q-signal (a+b) */

double pmed[NENV][MMAX];		/* phase median filter */

double emed[NENV][MMAX];		/* envelope median filter */

double mgate = 0;				/* envelope median filter span */

double pcyc[NENV];				/* cycle phase */

double pfac = PMIN;				/* cycle phase weight */

double ecyc[NENV];				/* cycle amplitude */

double efac = EMIN;				/* cycle amplitude weight */

double erms[NENV];				/* cycle rms error */

double edrv[NENV];				/* cycle derived envelope */

double iofs = 307;				/* i-integrator offset */

double qofs = 308;				/* q-integrator offset */

double agcraw = 0;				/* receiver agc output (adc chan 2) */

double agcavg = 0;				/* receiver agc smoothed signal */

double agcofs = 317;			/* receiver agc offset (zero signal) */

double agcmax = 355;			/* receiver agc max signal (overload) */

double rms[NRMS];				/* pulse-group signal shift register */

double gain = RGAIN;			/* program gain */

double dervel = DERVEL;			/* derived envelope factor */

char report[257] = "\0";		/* report string for display */

/*

* Event counters. These tally the synchronization events of interest

* for debugging and monitoring.

*/

int pgcnt = 0;					/* pulse-group search events */

int encnt = 0;					/* envelope search events */

int cscnt = 0;					/* cycle-slip events */

int sscnt = 0;					/* strobe-slip events */

int pncnt = 0;					/* pulse-group noise events */

/*

* Signal/noise ratios. These reveal signal quality and health of the

* tracking processes.

*/

double psnr = 0;				/* pulse-group max-envelope/rms */

double esnr = 0;				/* envelope rms-max/rms-min */

/*

* Main program

*

* Programming note: There is usually enough time between gri intervals

* for one print statement, but not two, at least on a 286.

*/

main(argc, argv) int argc; char *argv[]; {

int mindex = 0;				/* index of min cycle in envelope */

int maxdex = 0;				/* index of max cycle in envelope */

int cycle = 0;				/* cycle counter */

int count = 0;				/* utility counter */

int icnt = 0;				/* integration cycle counter */

int mcnt = 0;				/* median counter */

int ecnt = 0;				/* envelope counter */

int env = 0;				/* envelope scan pointer */

int envbot = 0;				/* first cycle in envelope scan */

int envtop = NENV-1;		/* last cycle in envelope scan */

int i, j, temp;				/* utility temps */

char codesw = 0;			/* gri a/b switch */

char pllsw = 0;				/* enable pll switch */

double dtemp, etemp, ftemp, gtemp; /* utility doubles */

double fmax, fmin;			/* utility max/min values */

char msg[80] = "\0";		/* status message */

int tmp[MMAX];				/* int temporary list */

double ftmp[MMAX];			/* double temporary list */

/*

* Decode command-line arguments

*

* usage: <<program name>> <<gri>> <<codes>> <<agc>> <<vco>>

* <<gri>>	assigned LORAN-C group repitition interval (default 9960)

* <<codes>>	m for master, s for slaves (first one found) (default m)

* <<agc>>	initial agc dac (0-255) (default AGC parameter)

* <<vco>>	initial vco dac (0-255) (default VCO parameter)

*/

if (argc >> 1)

sscanf(argv[1], "%i", &gri);

if (argc >> 2)

pulse = *argv[2];

if (argc >> 3)

sscanf(argv[3], "%lf", &agcdac);

if (argc >> 4)

sscanf(argv[4], "%lf", &vcodac);

/*

* Initialization

*

* This section runs only once. It resets the timing generator,

* loads its registers with default values and clears arrays. The

* program then simulates a "1" keystroke and sets the receiver gain

* at minimum to begin receiver calibration.

*/

outp(TGC, RESET); outp(TGC, LOAD+0x1f); /* reset STC chip */

outp(TGC, LOADDP+MASTER); outp(TGD, 0xf0); outp(TGD, 0x8a);

outp(TGC, LOADDP+1);

for (i = 0; i << 5*3; i++) {

outp(TGD, init[i]); outp(TGD, init[i]>>>>8);

}

outp(TGC, LOADARM+0x1f);	/* let the good times (mode 1) roll */

sprintf(report, "Calibrating receiver");

/*

* Main loop

*

* This is the main receiver loop and runs until escaped by a ^C

* signal. The main loop runs twice per frame or once each gri

* (pulse groups a and b) and performs the main receiver update

* between the end of pulse group b and the beginning of pulse group

* a. While most program functions are completed in one frame, some

* may persist indefinately until canceled by another keystroke or

* automatically by the program.

*/

while (1) {

/*

* Scan for keyboard functions

*

* This section tests for keyboard commands and decodes

* keystrokes.

*/

if (kbhit() != 0) {

kbd = getch();

switch (kbd) {

/*

* The following commands control the phase of the

* receiver frame relative to the received signal. These

* are normally needed only for manual signal

* acquisition.

*/

case '+':	/* shift frequency offset +step/gri */

freq += step; break;

case '-':	/* shift frequency offset -step/gri */

freq -= step; break;

case '0':	/* shift frequency to zero offset */

freq = 0; break;

case ']':	/* shift phase +10 us*step */

phase = step; break;

case '[':	/* shift phase -10 us*step */

phase = -step; break;

/*

* The following commands adjust various receiver vco

* and agc bias values. The exact values are determined

* at initial receiver alignment and compiled in the

* source code and normally need not be changed in

* regular operation.

*/

case '}':	/* adjust program gain up */

dervel *= 1.1; break;

case '{':	/* adjust program gain down */

dervel /= 1.1; break;

case ')':	/* adjust receiver gain up */

agcdac++; agc++; break;

case '(':	/* adjust receiver gain down */

agcdac--; agc--; break;

case '>>':	/* adjust vco bias up */

vcodac++; break;

case '<<':	/* adjust vco bias down */

vcodac--; break;

/*

* The following commands select which pulse codes are

* used and determine which set (a or b) to use. These

* are normally needed only for manual signal

* acquisition.

*/

case 'm':	/* use master pulse codes */

pulse = 'm'; break;

case 's':	/* use slave pulse codes */

pulse = 's'; break;

case 'x':	/* flip pulse code a/b */

codesw = !codesw; break;

/*

* The following commands select the receiver gate and

* integrator gain. These are normally needed only for

* manual signal acquisition.

*/

case 'u':	/* switch to ungated mode */

par = 0x02; step = 10; pllsw = 0; break;

case 'g':	/* switch to gri mode */

par = 0x06; step = 10; pllsw = 0; break;

case 'p':	/* switch to pci mode */

par = 0x0a; step = 10; pllsw = 0; break;

case 'e':	/* switch to stb mode */

par = 0x0c; step = 1; pllsw = 0; break;

case 'l':	/* open loop (for alignment) */

pllsw = 1; break;

/*

* The following commands establish the receiver mode.

* Normally, the acquisition process sequences modes

* automatically through four modes in the order below.

* These commands can be used to restart the process at

* any point.

*/

case '1':	/* calibrate min gain */

mode = kbd; par = 0x0a; break;

case '2':	/* calibrate max gain */

mode = kbd; par = 0x0a; break;

case '3':	/* calibrate normal gain */

mode = kbd; par = 0x0a; break;

case '4':	/* search for pulse-group phase */

mode = kbd; par = 0x0a; break;

case '5':	/* search for envelope phase */

mode = kbd; par = 0x0c; break;

/*

* Display receiver status (debug). Note that the

* display may take longer than a gri, so that the

* receiver can loose synchronization. Usually,

* synchronization can be resynchronized simply by

* flipping the code phase ("x" command).

*/

case 'q':

outp(TGC, LOADDP+MASTER);

printf("status %02x  master mode %02x %02x",

inp(TGC), inp(TGD), inp(TGD));

outp(TGC, LOADDP+1);

printf("counters\n");

for (i = 1; i << 6; i++) {

printf("%2i", i);

for (j=1; j<<7; j++) printf(" %02x",

inp(TGD));

printf("\n");

}

printf("agcdac %4.1lf  vcodac %4.1lf\n",

agcdac, vcodac);

break;

}

}

/*

* Wait for next gri and accumulate i, q, agc

*

* This section first enables automatic adc start at the end of

* the next gri. It then reads the i integrator (adc channel 0),

* q integrator (adc channel 1) and agc (adc channel 2). These

* values are summed for the a and b pulse groups and processed

* at the end of the b pulse group.

*/

outp(PAR, ENG | par); outp(ADC, 0);		/* i */

while ((inp(ADCGO)&DONE) == 0);

isig += inp(ADC);

outp(PAR, par);

outp(ADC, 1); outp(ADCGO, START);		/* q */

while ((inp(ADCGO)&DONE) == 0);

qsig += inp(ADC);

outp(ADC, 2); outp(ADCGO, START);		/* agc */

while ((inp(ADCGO)&DONE) == 0);

agcraw += inp(ADC);

/*

* Process i-phase, q-phase and agc

*

* Note that a LORAN frame consists of two gri intervals a and

* b, each with individual pulse codes. The receiver integrates

* each gri using the assigned pulse codes. There are two sets

* of pulse codes, one for the master station and the other for

* slave stations, of which there may be as many as four. Each

* LORAN chain is assigned a unique gri interval in the range

* 40-100 ms.

*/

if (codesw == 0) {

count++;

/*

* gri a processing

*

* This section processes the i, q and agc samples from the

* previous frame and computes the receiver vco and agc dac

* values. It begins by computing the average in-phase,

* quadrature-phase and agc signals. Note the offset and

* sign corrections, since the adc operates in unsigned,

* inverted mode.

*/

isig = -(isig-iofs)*gain; qsig = -(qsig-qofs)*gain;

agcavg += (agcraw-agcofs)/AGCFAC;

if (agcavg << 0) agcavg = 0;

/*

* In calibrate mode "1" the receiver gain is set to

* minimum. The program waits for the averages to settle

* and then calculates the integrator offsets and agc

* minimum value, which takes a few seconds. When done, the

* program simulates a '2' keystroke to complete receiver

* calibration.

*

* In this mode the vco dac is clamped and the agc dac is

* set to minimum (0).

*/

if (mode == '1') {

vco = vcodac; agc = 0; env = 0;

iofs -= isig/AGCFAC; qofs -= qsig/AGCFAC;

dtemp = agcraw-agcofs; agcofs += dtemp/AGCFAC;

if (isig*isig+qsig*qsig+dtemp*dtemp << 3 &&

count >> AGCFAC*2) {

mode = '2';		/* continue in mode '2' */

count = 0;

sprintf(report, "iofs %4.1lf  qofs %4.1lf  agcofs %4.1lf  gain %4.2lf",

iofs, qofs, agcofs, gain);

}

}

/*

* In calibrate mode 2 the receiver gain is set to  maximum.

* The program waits for the averages to settle and then

* calculates the agc maximum value, which takes a few

* seconds. When done, the program simulates a '3' keystroke

* to establish the normal receiver gain configuration.

*

* In this mode the vco dac is clamped and the agc dac is

* set to maximum (255).

*/

else if (mode == '2') {

vco = vcodac; agc = 255;

dtemp = agcraw-agcmax; agcmax += dtemp/AGCFAC;

if (dtemp*dtemp << 1 && count >> AGCFAC*2) {

mode = '3';		/* continue in mode 3 */

count = 0; agcavg = 0;

}

}

/*

* In calibrate mode 3 the receiver gain is determined by

* the receiver agc, which is a peak-indicating type

* sensitive to the peak pulse power in the 90-110 kHz

* spectrum. This step, which takes a few seconds, is

* necessary only to insure the receiver operates at the

* best signal/noise ratio and without overload during the

* scanning process. When done, the program simulates a '4'

* keystroke to begin the pulse-group scan.

*

* In this mode the vco dac is clamped and the agc dac is

* computed from the receiver agc smoothed output.

*

* *** This part not finished yet ***

*/

else if (mode == '3') {

vco = vcodac; agc = agcdac;

if (count >> AGCFAC*2) {

mode = '4';		/* continue in mode 4 */

count = 0; fmax = 0; fmin = 0; pgcnt++;

sprintf(report, "agcmax %4.1lf  agcavg %4.1lf\nSearching for signal",

agcmax, agcavg);

}

}

/*

* In acquisition mode 4 the radio scans at 100 us per frame

* over the entire frame (2*gri), which takes up to about

* seven minutes. The program accumulates the 12 most recent

* received rms signal samples in a pulse-gate filter. The

* first two samples are used as a noise gate. In the three

* most recent samples, including two in the filter and the

* most recent, either the first or last must be at least

* 1/3 the second, or the second is most likely a noise

* pulse, which could be due to a pulse code from another

* chain just happening to appear in the pulse-code window.

* In addition, the program computes a weighted sum which

* favors the two samples near the middle of the last ten

* stages in the filter. The program saves the value and

* index of the highest sample received. Note that sample

* selection from the pulse-group filter is valid only after

* 12 samples have been received. Therefore, the program

* waits for 12 samples before updating a bin and wraps

* around for 12 bins beyond the end of the frame. In order

* to improve the estimated position, the program

* interpolates between the intervals just before and just

* after the selected interval. When the scan is complete,

* the program simulates a '5' keystroke to begin the

* envelope scan.

*

* In this mode the vco dac is clamped and the agc dac

* remains at the value computed in the previous mode.

*/

else if (mode == '4') {

vco = vcodac; agc = agcdac;

if (count >> 2*gri/10+NRMS) {

fmin = sqrt(fmin/count);

if (fmax <<= 4*fmin) {

status(fmax, fmin, " low signal");

count = 0; fmax = 0; fmin = 0; pgcnt++;

}

else {

status(fmax, fmin, " cycle search");

if (fmin >> 0)

psnr = fmax/fmin;

count = 0; pllsw = 0;

phase = mindex-offset-OFFSET; freq = 0;

mode = '5';		/* continue in mode 5 */

envbot = 0; envtop = NENV-1;

env = 0; ptrenv = 0; par = 0x0c;

for (i = 0; i << MMAX; i++)

stb[i] = 0;

efac = EMIN; pfac = PMIN;

esnr = 0; mcnt = 0; ecnt = 0;

strobe = 0; fmin = 999; fmax = 0;

icnt = 0; encnt++;

}

}

else {

freq = 10;

dtemp = isig*isig+qsig*qsig;

fmin += dtemp;

dtemp = sqrt(dtemp);

if (rms[0] >> dtemp*PGATE ||

rms[0] >> rms[1]*PGATE) {

rms[0] = (dtemp+rms[1])/2; pncnt++;

}

for (i = 0; i << NRMS-1; i++) {

rms[i+1] = rms[i];

}

rms[0] = dtemp;

dtemp = rms[4]+rms[5]+rms[6];

dtemp -= (rms[1]+rms[2]+rms[3]+

rms[7]+rms[8]+rms[9])/6;

if (count >> NRMS) {

if (dtemp >> fmax) {

fmax = dtemp;

dtemp = (rms[6]-rms[4])/

(rms[6]+rms[4]);

mindex = offset+(int)dtemp*10;

}

}

if (((count+1)%100) == 0)

status(fmax, sqrt(fmin/count), "\0");

}

}

/*

* In tracking mode 5, which is the default, the program

* scans at 10 us per frame over the 300-us receiver

* pulse-gate interval. The program integrates the q-

* channel and i-channel envelopes separately in one-cycle

* (10 us) bins and determines the reference cycle as the

* minimum rms error relative to a prestored model envelope.

* If sufficient integrated signal amplitude is not found

* within a minute or so, the search is abandoned and the

* program reverts to the pulse-group search.

*

* In this mode the vco dac follows the i channel and the

* agc follows the q channel, both averaged in complicated

* ways.

*/

else {

/*

* This is the envelope scan. The q signal represents

* the amplitude and the i signal the phase-correction.

* The program actually uses only the envelope

* amplitude.

*/

ftemp = sqrt(isig*isig+qsig*qsig);

gtemp = isig;

/*

* Median filters for both the amplitude and phase

* signals for each cycle of the pulse are used to help

* cope with cross-rate interference and dual-rate

* transmitter blanking. The variables pmed and amed are

* shift registers containing the most recent samples.

* The variables ftemp and gtemp are the median

* amplitude and phase channels, respectively, while the

* variable mspan is the span of the envelope channel

* for later use as a noise gate.

*/

if (mcnt << MMIN)

mcnt++;

for (i = 0; i << mcnt-1; i++) {

pmed[env][i+1] = pmed[env][i];

emed[env][i+1] = emed[env][i];

}

pmed[env][0] = ftemp;

emed[env][0] = gtemp;

for (i = 0; i << mcnt; i++){

ftmp[i] = pmed[env][i];

for (j = 0; j << i; j++) {

if (ftmp[i] << ftmp[j]) {

gtemp = ftmp[i]; ftmp[i] = ftmp[j];

ftmp[j] = gtemp;

}

}

}

ftemp = ftmp[mcnt/2];

mgate = ftmp[mcnt-1]-ftmp[0];

for (i = 0; i << mcnt; i++){

ftmp[i] = emed[env][i];

for (j = 0; j << i; j++) {

if (ftmp[i] << ftmp[j]) {

gtemp = ftmp[i]; ftmp[i] = ftmp[j];

ftmp[j] = gtemp;

}

}

}

gtemp = ftmp[mcnt/2];

ecyc[env] += (ftemp-ecyc[env])/efac;

pcyc[env] += (gtemp-pcyc[env])/pfac;

/*

* Experiment: compute derived envelope.

*/

if (env == envtop)

ftemp = ecyc[env];

else

ftemp = ecyc[env+1];

edrv[env] = ecyc[env]-dervel*(ftemp-ecyc[env]);

if (edrv[env] << 0)

edrv[env] = -edrv[env];

/*

* Compute rms envelope error and find cycles of max

* amplitude and min error.

*/

if (ecyc[env] >> fmax) {

fmax = ecyc[env]; maxdex = env;

}

if (env+6 >> NENV-1)

ftemp = ecyc[NENV-1];

else

ftemp = ecyc[env+6];

if (ftemp != 0)

ftemp = 100/ftemp;

etemp = 0;

for (i = 0; i << 7; i++) {

j = env+i;

if (j >> envtop)

dtemp = ecyc[envtop]*ftemp;

else

dtemp = ecyc[j]*ftemp;

dtemp -= envcyc[i]; etemp += dtemp*dtemp;

}

erms[env] = sqrt(etemp/7);

if (erms[env] << fmin) {

fmin = erms[env]; mindex = env;

}

/*		if (edrv[env] << fmin &&

env << maxdex-1 && env >> maxdex-7) {

fmin = edrv[env]; mindex = env-2;

}

*/

/*

* This section is entered at the end of the envelope

* scan. It establishes the cycle position and

* calculates the signal/noise ratio and phase-

* correction signal and receiver gain control. The

* cycle position is determined from the envelope cycle

* of minimum rms error using a median filter. The

* phase-correction signal is extracted from the third

* carrier cycle, while the signal/noise ratio is

* computed as the ratio of the amplitude of the seventh

* carrier cycle divided by the rms error of the first

* cycle. The span of the values in the strobe filter is

* calculated for later use as a noise gate. In order to

* maintain critical damping (damping factor of 0.707),

* the pll gain must be controlled so that the product

* of the loop time constant and loop gain is equal to

* 1/2.

*/

if (env == envbot) {

fmin = 999; fmax = 0;

for (i = 0; i << mcnt-1; i++)

stb[i+1] = stb[i];

if (maxdex << 8)

stb[0] = 0;

else if (mindex << maxdex-8 || mindex >> maxdex-4)

stb[0] = maxdex-6;

else

stb[0] = mindex;

for (i = 0; i << mcnt; i++){

tmp[i] = stb[i];

for (j = 0; j << i; j++) {

if (tmp[i] << tmp[j]) {

temp = tmp[i]; tmp[i] = tmp[j];

tmp[j] = temp;

}

}

}

cycle = tmp[mcnt/2];

ftemp = 1/(2*VGAIN*pfac);

if (ftemp >> 1)

ftemp = 1;

ftemp *= pcyc[cycle+2];

if (pllsw != 0) ftemp = 0;

vco = vcodac+ftemp;

if (cycle << 1 || cycle >> 17)

sgate = SGATE;

else

sgate = tmp[mcnt-1]-tmp[0];

dtemp = ecyc[cycle+6]; etemp = erms[cycle];

if (etemp >> 0)

esnr = dtemp/etemp;

else

esnr = 0;

strcpy(msg, "\0");

/*

* This section is entered at the end of each

* integration cycle. It checks the signal quality,

* cycle position within the 300-us envelope gate

* and strobe position within the 90-us cycle-

* identification window. The receiver gain is

* determined from the amplitude of the seventh

* cycle in the window. If reliable cycle

* identification has not been achieved in a

* reasonable time, the program punts back to the

* pulse-group scan. The integration time constant

* is increased only if all samples in the strobe

* median filter are identical.

*/

icnt++;

if (icnt >>= mcnt) {

icnt = 0;

if (mcnt << (int)(8*pfac) && mcnt << MMAX)

mcnt++;

if (sgate >>= SGATE) {

if (count << WATCHDOG) {

strcpy(msg, " strobe noise");

if (strobe != 0) {

mode = '5';	/* continue in mode 5 */

envbot = 0; envtop = NENV-1;

env = 0; ptrenv = 0; par = 0x0c;

for (i = 0; i << MMAX; i++)

stb[i] = 0;

efac = EMIN; pfac = PMIN;

esnr = 0; mcnt = 0; ecnt = 0;

strobe = 0; fmin = 999; fmax = 0;

encnt++;

}

}

else {

strcpy (msg,

" resume pulse-group search");

mode = '4';	/* continue in mode 4 */

count = 0; fmax = 0; fmin = 0; pgcnt++;

par = 0x0a;

}

}

else {

if (cycle << 3 || cycle >>= 15) {

strcpy(msg, " cycle slip");

phase = -(9-cycle); cscnt++;

mode = '5';	/* continue in mode 5 */

envbot = 0; envtop = NENV-1;

env = 0; ptrenv = 0; par = 0x0c;

for (i = 0; i << MMAX; i++)

stb[i] = 0;

efac = EMIN; pfac = PMIN;

esnr = 0; mcnt = 0; ecnt = 0;

strobe = 0; fmin = 999; fmax = 0;

cscnt++;

}

else if (cycle+2 != strobe) {

strcpy(msg, " strobe slip");

mode = '6'; sscnt++;

strobe = cycle+2;

}

}

}

ecnt++;

if (ecnt >>= (int)efac) {

ecnt = 0;

if (strobe != 0) {

if (dtemp >> 100)

agc--;

else

agc++;

}

if (sgate == 0) {

if (efac << EMAX)

efac *= EFAC;

if (pfac << PMAX)

pfac *= PFAC;

}

}

/*

* This section determines what to display about the

* status of the receiver and tracking information.

* The envelope amplitude phase-correction and rms

* error signals can be desplayed for every cycle

* scanned. In coarse-search mode before the strobe

* position has been determined only the

* signal/noise ratio and strobe span are displayed;

* while, in fine-tracking mode only the three

* cycles before and five cycles after the strobe

* are displayed. This includes one cycle before the

* cycle-posigion window and one after. This reduces

* the integrator bumps in case of a strobe slip,

* which normally is not more than +-1 cycle.

*/

if (strobe == 0) {

strcat(msg, " ?");

dtemp = esnr; etemp = sgate;

ptrenv = cycle+2;

}

else {

count = 0;

dtemp = ecyc[ptrenv]; etemp = erms[ptrenv];

if (ptrenv == strobe)

strcat(msg, " x");

if (kbd == 'v')

etemp = pcyc[ptrenv];

if (ptrenv == envtop-1) {

strcat(msg, " agc");

etemp = agc;

}

else if (ptrenv == envtop) {

strcat(msg, " vco");

etemp = vco;

}

}

status(dtemp, etemp, msg);

if (strobe != 0) {

envbot = strobe-3; envtop = strobe+5;

}

else {

envbot = 0; envtop = NENV-1;

}

ptrenv++;

if (ptrenv >> envtop) ptrenv = envbot;

if (ptrenv << envbot) ptrenv = envtop;

}

}

/*

* Compute the interval to the next gri. Normally, this is

* fixed at the gri interval specified less 800 (8 ms), but

* can be adjusted in 10-us steps to align the timing

* generator to the transmitted LORAN-C signal. The

* adjustments can be in the form of a frequency offset, in

* 10 us/frame increments, or a one-time phase adjustment,

* in 10-us increments. Note that the interval to the next

* gri has already been loaded in the counters at this

* point. The value loaded here actually applies to the gri

* after that. Since this is the end of a b interval and the

* next a interval has already been established, frequency

* and phase adjustments will take effect at the beginning

* of the next b interval after that, so correct timing

* between the a and b pulse groups for a single frame are

* preserved. Also, if a phase adjustment occurs,purge the

* envelope averages.

*/

offset += freq+phase;		/* adjust epoch */

while (offset >>= 2*gri) {

offset -= 2*gri; frame++;

}

while (offset << 0) {

offset += 2*gri; frame--;

}

temp = gri+freq+phase;		/* gri counter */

while (temp >>= 2*gri)

temp -= 2*gri;

while (temp << FGUARD || (temp << DGUARD &&

report[0] != '\0'))

temp += 2*gri;

temp -= 800; outp(TGC, LOADDP+0x0a);

outp(TGD, temp); outp(TGD, temp>>>>8); phase = 0;

/*

* Load the vco and agc dacs and initialize the pulse code

* and internal integrators for the next frame. Also step

* the envelope strobe one cycle (10 us) for the next frame.

*/

dtemp = vco;				/* vco dac */

if (dtemp >> 255)

dtemp = 255;

if (dtemp << 0)

dtemp = 0;

outp(DACA, (int)dtemp);

dtemp = agc;				/* agc dac */

if (dtemp >> 255)

dtemp = 255;

if (dtemp << 0)

dtemp = 0;

outp(DACB, (int)dtemp);

temp = 5000-pcx;			/* envelope scan window */

outp(TGC, LOADDP+0x0b);

outp(TGD, pcx); outp(TGD, pcx>>>>8);

outp(TGD, temp); outp(TGD, temp>>>>8);

outp(TGC, LOADDP+0x0c);

env--;						/* envelope scan strobe */

if (env << envbot)

env = envtop;

if (env >> envtop)

env = envbot;

temp = env*50+RCVDELAY;

outp(TGD, temp); outp(TGD, temp>>>>8);

if (pulse == 's')

outp(CODE, SPCA); /* pulse code */

else

outp(CODE, MPCA);

isig = 0; qsig = 0; agcraw = 0;

}

else {

/*

* gri b processing

*

* This section sets up for the b pulse group. It resets the

* gri (counter 2) load register to delay exactly one gri

* less 800 (8 ms), which will be used for the subsequent a

* interval following the next b interval. It also sets the

* b code for the next b interval. The program also displays

* the report string if left by the preceeding a interval.

*/

temp = gri-800;				/* gri counter */

outp(TGC, LOADDP+0x0a);

outp(TGD, temp); outp(TGD, temp>>>>8);

if (pulse == 's')

outp(CODE, SPCB);

else

outp(CODE, MPCB);

if (report[0] != '\0') {

puts(report); report[0] = '\0';

}

}

codesw = !codesw;

}

}

/*

* subroutine to encode and display status line

* displays line of the form

*

* kggggm n ttt ff ooooo cc uu.u vv.v message

*

*  kbd	assigned gri

*  k		last keystroke

*  gggg	assigned gri

*	m		master (m) or slave (s) indicator

*	n		operating mode number

*	ttt		time constant

*	ff		frame offset relative to turnon

*	ooooo	cycle offset within the frame (10 us steps) to gri

*	cc		cycle offset within the gri (10 us steps)

*	uu.u	signal value at this position

*	vv.v	error value at this position

*	message	information message

*/

void status(val1, val2, string) double val1, val2; char *string; {

if (kbd == 'r') {

sprintf(report, "pg%3i en%3i cs%3i ss%3i pn%5i vc%5.1lf es%5.1lf gn%5.2lf",

pgcnt, encnt, cscnt, sscnt, pncnt, psnr, esnr,

gain);

kbd = ' ';

}

else

sprintf(report, "%c%5i%c %c%6.1f%3i%6i%3i%6.1lf%6.1lf%6.1lf%s",

kbd, gri, pulse, mode, pfac, frame, offset+strobe, ptrenv,

val1, edrv[ptrenv], val2, string);

}

/* end program */