@PARAFILTR ON =

@SUBHEAD = Abstract

This document describes the Network Time Protocol (NTP), specifies its formal structure and summarizes information useful for its implementation. NTP provides the mechanisms to synchronize time and coordinate time distribution in a large, diverse internet operating at rates from mundane to lightwave. It uses a returnable-time design in which a distributed subnet of time servers operating in a self-organizing, hierarchical-master-slave configuration synchronizes local clocks within the subnet and to national time standards via wire or radio. The servers can also redistribute reference time via local routing algorithms and time daemons.

This is an Internet Standard Recommended Protocol with assigned number RFC-1119.

Keywords: network clock synchronization, standard time distribution, fault-tolerant architecture, maximum-likelihood estimation, disciplined oscillator, internet protocol, formal specification.

Sponsored by: Defense Advanced Research Projects Agency contract number N00140-87-C-8901 and by National Science Foundation grant number NCR-86-12015.

@PAGE =

@Z_TOC TITLE = Table of Contents

@Z_TOC LVL 1 = 1.	 Introduction	1

@Z_TOC LVL 2 = 1.1.	 Related Technology	2

@Z_TOC LVL 1 = 2.	 System Architecture	3

@Z_TOC LVL 2 = 2.1.	 Implementation Model	4

@Z_TOC LVL 2 = 2.2.	 Network Configurations	5

@Z_TOC LVL 2 = 2.3.	 The NTP Timescale	6

@Z_TOC LVL 2 = 2.4.	 The NTP Calendar	8

@Z_TOC LVL 2 = 2.5.	 Time and Frequency Dissemination	9

@Z_TOC LVL 1 = 3.	 Network Time Protocol	10

@Z_TOC LVL 2 = 3.1.	 Data Formats	10

@Z_TOC LVL 2 = 3.2.	 State Variables and Parameters	11

@Z_TOC LVL 3 = 3.2.1.	 Common Variables	11

@Z_TOC LVL 3 = 3.2.2.	 System Variables	13

@Z_TOC LVL 3 = 3.2.3.	 Peer Variables	13

@Z_TOC LVL 3 = 3.2.4.	 Packet Variables	14

@Z_TOC LVL 3 = 3.2.5.	 Clock Filter Variables	15

@Z_TOC LVL 3 = 3.2.6.	 Parameters	17

@Z_TOC LVL 2 = 3.3.	 Modes of Operation	18

@Z_TOC LVL 2 = 3.4.	 Event Processing	20

@Z_TOC LVL 3 = 3.4.1.	 Transmit Procedure	20

@Z_TOC LVL 3 = 3.4.2.	 Receive Procedure	21

@Z_TOC LVL 3 = 3.4.3.	 Packet Procedure	23

@Z_TOC LVL 3 = 3.4.4.	 Primary-clock procedure	25

@Z_TOC LVL 3 = 3.4.5.	 Clock-update procedure	25

@Z_TOC LVL 3 = 3.4.6.	 Initialization Procedure	26

@Z_TOC LVL 3 = 3.4.7.	 Clear Procedure	27

@Z_TOC LVL 3 = 3.4.8.	 Poll-update procedure	28

@Z_TOC LVL 2 = 3.5.	 Access Control Issues	28

@Z_TOC LVL 1 = 4.	 Filtering and Selection Algorithms	29

@Z_TOC LVL 2 = 4.1.	 Clock-filter procedure	29

@Z_TOC LVL 2 = 4.2.	 Clock-selection procedure	30

@Z_TOC LVL 1 = 5.	 Local Clocks	31

@Z_TOC LVL 2 = 5.1.	 Standard Oscillators	32

@Z_TOC LVL 2 = 5.2.	 Mathematical Model	33

@Z_TOC LVL 2 = 5.3.	 Fuzzball Implementation	35

@Z_TOC LVL 2 = 5.4.	 Uniform Phase Adjustments	36

@Z_TOC LVL 2 = 5.5.	 Nonuniform Phase Adjustments	37

@Z_TOC LVL 2 = 5.6.	 Maintaining Date and Time	37

@Z_TOC LVL 1 = 6.	 Acknowledgments	38

@Z_TOC LVL 1 = 7.	 References	38

@Z_TOC LVL 1 = 8.	 Appendix A. NTP Data Format - Version 2	42

@Z_TOC LVL 1 = 9.	 Appendix B. NTP Control Messages	45

@Z_TOC LVL 2 = 9.1.	 NTP Control Message Format	46

@Z_TOC LVL 2 = 9.2.	 Status Words	47

@Z_TOC LVL 3 = 9.2.1.	 System Status Word	48

@Z_TOC LVL 3 = 9.2.2.	 Peer Status Word	49

@Z_TOC LVL 3 = 9.2.3.	 Clock Status Word	49

@Z_TOC LVL 3 = 9.2.4.	 Error Status Word	50

@Z_TOC LVL 2 = 9.3.	 Commands	50

@Z_TOC LVL 1 = 10.	 Appendix C. Authentication Issues	53

@Z_TOC LVL 2 = 10.1.	 NTP Authentication Mechanism	54

@Z_TOC LVL 2 = 10.2.	 NTP Authentication Procedures	55

@Z_TOC LVL 1 = 11.	 Appendix D. Differences from Previous Versions.	57

@Z_TOC TITLE = List of Figures

@Z_TOC LVL 4 = Figure 1. Implementation Model	4

@Z_TOC LVL 4 = Figure 2. Calculating Delay and Offset	24

@Z_TOC LVL 4 = Figure 3. Phase-Lock Loop Model	33

@Z_TOC LVL 4 = Figure 4. Clock Registers	35

@Z_TOC LVL 4 = Figure 5. NTP Message Header	42

@Z_TOC LVL 4 = Figure 6. NTP Control Message Header	46

@Z_TOC LVL 4 = Figure 7. Status Word Formats	47

@Z_TOC LVL 4 = Figure 8. Authenticator Format	54

@Z_TOC TITLE = List of Tables

@Z_TOC LVL 4 = Table 1. Dates of Leap-Second Insertion	7

@Z_TOC LVL 4 = Table 2. System Variables	14

@Z_TOC LVL 4 = Table 3. Peer Variables	15

@Z_TOC LVL 4 = Table 4. Packet Variables	16

@Z_TOC LVL 4 = Table 5. Parameters	16

@Z_TOC LVL 4 = Table 6. Modes and Actions	22

@Z_TOC LVL 4 = Table 7. Characteristics of Standard Oscillators	32

@Z_TOC LVL 4 = Table 8. Clock Parameters	35