@PARAFILTR ON =

@Z_TOC TITLE = Abstract

This document describes the Network Time Protocol (NTP), specifies its formal structure and summarizes information useful for its implementation. NTP provides the mechanisms to synchronize time and coordinate time distribution in a large, diverse internet operating at rates from mundane to lightwave. It uses a returnable-time design in which a distributed subnet of time servers operating in a self-organizing, hierarchical-master-slave configuration synchronizes local clocks within the subnet and to national time standards via wire or radio. The servers can also redistribute reference time via local routing algorithms and time daemons.

This is an Internet Standard Recommended Protocol. Distribution of this memo is unlimited.

Keywords: network clock synchronization, standard time distribution, fault-tolerant architecture, maximum-likelihood estimation, disciplined oscillator, internet protocol, high-speed networks, formal specification.

Sponsored by: Defense Advanced Research Projects Agency contract number N00140-87-C-8901, NASA Ames Research Center contract number NAG 2-638 and National Science Foundation grant number NCR-89-13623.

@PAGE =

@Z_TOC TITLE = Preface

This document describes Version 3 of the Network Time Protocol (NTP). It supersedes Version 2 of the protocol described in RFC-1119 dated September 1989. However, it neither changes the protocol in any significant way nor obsoletes previous versions or existing implementations. The main motivation for the mew version is to refine the analysis and implementation models for new applications at much higher network speeds to the gigabit-per-second regime and to provide for the enhanced stability, accuracy and precision required at such speeds. In particular, the sources of time and frequency errors have been rigorously examined and error bounds established in order to improve performance, provide a model for correctness assertions and indicate timekeeping quality to the user. The revision also incorporates two new optional features, (1) an algorithm to combine the offsets of a number of peer time servers in order to enhance accuracy and (2) improved local-clock algorithms which allow the poll intervals on all synchronization paths to be substantially increased in order to reduce network overhead. An overview of the changes, which are described in detail in Appendix D, follows:

@INDENT HEAD = 1.

@INDENT = In Version 3 The local-clock algorithm has been overhauled to improve stability and accuracy. Appendix G presents a detailed mathematical model and design example which has been refined with the aid of feedback-control analysis and extensive simulation using data collected over ordinary Internet paths. Section 5 of RFC-1119 on the NTP local clock has been completely rewritten to describe the new algorithm. Since the new algorithm can result in message rates far below the old ones, it is highly recommended that they be used in new implementations. Note that this algorithm is not integral to the NTP protocol specification itself and its use does not affect interoperability with previous versions or existing implementations; however, in order to insure overall NTP subnet stability in the Internet, it is essential that the local-clock characteristics of all NTP time servers conform to the analytical models presented previously and in this document.

@INDENT HEAD = 2.

@INDENT = In Version 3 a new algorithm to combine the offsets of a number of peer time servers is presented in Appendix F. This algorithm is modelled on those used by national standards laboratories to combine the weighted offsets from a number of standard clocks to construct a synthetic laboratory timescale more accurate than that of any clock separately. It can be used in an NTP implementation to improve accuracy and stability and reduce errors due to asymmetric paths in the Internet. The new algorithm has been simulated using data collected over ordinary Internet paths and, along with the new local-clock algorithm, implemented and tested in the Fuzzball time servers now running in the Internet. Note that this algorithm is not integral to the NTP protocol specification itself and its use does not affect interoperability with previous versions or existing implementations.

@INDENT HEAD = 3.

@INDENT = Several inconsistencies and minor errors in previous versions have been corrected in Version 3. The description of the procedures has been rewritten in pseudo-code augmented by English commentary for clarity and to avoid ambiguity. Appendix I has been added to illustrate C-language implementations of the various filtering and selection algorithms suggested for NTP. Additional information is included in Section 5 and in Appendix E, which includes the tutorial material formerly included in Section 2 of RFC-1119, as well as much new material clarifying the interpretation of timescales and leap seconds.

@INDENT HEAD = 4.

@INDENT = Minor changes have been made in the Version-3 local-clock algorithms to avoid problems observed when leap seconds are introduced in the UTC timescale and also to support an auxiliary precision oscillator, such as a cesium clock or timing receiver, as a precision timebase. In addition, changes were made to some procedures described in Section 3 and in the clock-filter and clock-selection procedures described in Section 4. While these changes were made to correct minor bugs found as the result of experience and are recommended for new implementations, they do not affect interoperability with previous versions or existing implementations in other than minor ways (at least until the next leap second).

@INDENT HEAD = 5.

@INDENT = In Version 3 changes were made to the way delay, offset and dispersion are defined, calculated and processed in order to reliably bound the errors inherent in the time-transfer procedures. In particular, the error accumulations were moved from the delay computation to the dispersion computation and both included in the clock filter and selection procedures. The clock-selection procedure was modified to remove the first of the two sorting/discarding steps and replace with an algorithm first proposed by Marzullo and later incorporated in the Digital Time Service. These changes do not significantly affect the ordinary operation of or compatibility with various versions of NTP, but they do provide the basis for formal statements of correctness as described in Appendix H.

@PAGE =

@Z_TOC TITLE = Table of Contents

@Z_TOC LVL 1 = 1.	 Introduction	1

@Z_TOC LVL 2 = 1.1.	 Related Technology	2

@Z_TOC LVL 1 = 2.	 System Architecture	4

@Z_TOC LVL 2 = 2.1.	 Implementation Model	5

@Z_TOC LVL 2 = 2.2.	 Network Configurations	6

@Z_TOC LVL 1 = 3.	 Network Time Protocol	8

@Z_TOC LVL 2 = 3.1.	 Data Formats	8

@Z_TOC LVL 2 = 3.2.	 State Variables and Parameters	8

@Z_TOC LVL 3 = 3.2.1.	 Common Variables	9

@Z_TOC LVL 3 = 3.2.2.	 System Variables	11

@Z_TOC LVL 3 = 3.2.3.	 Peer Variables	11

@Z_TOC LVL 3 = 3.2.4.	 Packet Variables	13

@Z_TOC LVL 3 = 3.2.5.	 Clock-Filter Variables	13

@Z_TOC LVL 3 = 3.2.6.	 Authentication Variables	14

@Z_TOC LVL 3 = 3.2.7.	 Parameters	15

@Z_TOC LVL 2 = 3.3.	 Modes of Operation	16

@Z_TOC LVL 2 = 3.4.	 Event Processing	18

@Z_TOC LVL 3 = 3.4.1.	 Notation Conventions	18

@Z_TOC LVL 3 = 3.4.2.	 Transmit Procedure	19

@Z_TOC LVL 3 = 3.4.3.	 Receive Procedure	21

@Z_TOC LVL 3 = 3.4.4.	 Packet Procedure	23

@Z_TOC LVL 3 = 3.4.5.	 Clock-Update Procedure	25

@Z_TOC LVL 3 = 3.4.6.	 Primary-Clock Procedure	26

@Z_TOC LVL 3 = 3.4.7.	 Initialization Procedures	27

@Z_TOC LVL 4 = 3.4.7.1.	 Initialization Procedure	27

@Z_TOC LVL 4 = 3.4.7.2.	 Initialization-Instantiation Procedure	27

@Z_TOC LVL 4 = 3.4.7.3.	 Receive-Instantiation Procedure	28

@Z_TOC LVL 4 = 3.4.7.4.	 Primary Clock-Instantiation Procedure	29

@Z_TOC LVL 3 = 3.4.8.	 Clear Procedure	29

@Z_TOC LVL 3 = 3.4.9.	 Poll-Update Procedure	30

@Z_TOC LVL 2 = 3.5.	 Synchronization Distance Procedure	30

@Z_TOC LVL 2 = 3.6.	 Access Control Issues	31

@Z_TOC LVL 1 = 4.	 Filtering and Selection Algorithms	31

@Z_TOC LVL 2 = 4.1.	 Clock-Filter Procedure	32

@Z_TOC LVL 2 = 4.2.	 Clock-Selection Procedure	33

@Z_TOC LVL 1 = 5.	 Local Clocks	36

@Z_TOC LVL 2 = 5.1.	 Fuzzball Implementation	37

@Z_TOC LVL 2 = 5.2.	 Gradual Phase Adjustments	38

@Z_TOC LVL 2 = 5.3.	 Step Phase Adjustments	40

@Z_TOC LVL 2 = 5.4.	 Implementation Issues	41

@Z_TOC LVL 1 = 6.	 Acknowledgments	42

@Z_TOC LVL 1 = 7.	 References	42

@Z_TOC LVL 1 = A.	 Appendix A. NTP Data Format - Version 3	46

@Z_TOC LVL 1 = B.	 Appendix B. NTP Control Messages	49

@Z_TOC LVL 2 = B.1.	 NTP Control Message Format	50

@Z_TOC LVL 2 = B.2.	 Status Words	51

@Z_TOC LVL 3 = B.2.1.	 System Status Word	52

@Z_TOC LVL 3 = B.2.2.	 Peer Status Word	53

@Z_TOC LVL 3 = B.2.3.	 Clock Status Word	53

@Z_TOC LVL 3 = B.2.4.	 Error Status Word	54

@Z_TOC LVL 2 = B.3.	 Commands	54

@Z_TOC LVL 1 = C.	 Appendix C. Authentication Issues	57

@Z_TOC LVL 2 = C.1.	 NTP Authentication Mechanism	58

@Z_TOC LVL 2 = C.2.	 NTP Authentication Procedures	59

@Z_TOC LVL 3 = C.2.1.	 Encrypt Procedure	59

@Z_TOC LVL 3 = C.2.2.	 Decrypt Procedure	60

@Z_TOC LVL 3 = C.2.3.	 Control-Message Procedures	60

@Z_TOC LVL 1 = D.	 Appendix D. Differences from Previous Versions.	61

@Z_TOC LVL 1 = E.	 Appendix E. The NTP Timescale and its Chronometry	64

@Z_TOC LVL 2 = E.1.	 Introduction	64

@Z_TOC LVL 2 = E.2.	 Primary Frequency and Time Standards	65

@Z_TOC LVL 2 = E.3.	 Time and Frequency Dissemination	66

@Z_TOC LVL 2 = E.4.	 Determination of Frequency	68

@Z_TOC LVL 2 = E.5.	 Calendar Systems	68

@Z_TOC LVL 2 = E.6.	 The Modified Julian Day System	70

@Z_TOC LVL 2 = E.7.	 Determination of Time and Leap Seconds	70

@Z_TOC LVL 2 = E.8.	 The NTP Timescale and Reckoning with UTC	71

@Z_TOC LVL 1 = F.	 Appendix F. The NTP Clock-Combining Algorithm	73

@Z_TOC LVL 2 = F.1.	 Introduction	73

@Z_TOC LVL 2 = F.2.	 Determining Time and Frequency	73

@Z_TOC LVL 2 = F.3.	 Clock Modelling	73

@Z_TOC LVL 2 = F.4.	 Development of a Composite Timescale	74

@Z_TOC LVL 2 = F.5.	 Application to NTP	76

@Z_TOC LVL 2 = F.6.	 Clock-Combining Procedure	77

@Z_TOC LVL 1 = G.	 Appendix G. NTP Phase-Lock Loop Analysis	78

@Z_TOC LVL 2 = G.1.	 Mathematical Model	78

@Z_TOC LVL 2 = G.2.	 Implementation	80

@Z_TOC LVL 1 = H.	 Appendix H. Analysis of Errors and Correctness Principles	83

@Z_TOC LVL 2 = H.1.	 Introduction	83

@Z_TOC LVL 2 = H.2.	 Timestamp Errors	83

@Z_TOC LVL 2 = H.3.	 Measurement Errors	85

@Z_TOC LVL 2 = H.4.	 Network Errors	86

@Z_TOC LVL 2 = H.5.	 Inherited Errors	87

@Z_TOC LVL 2 = H.6.	 Correctness Principles	89

@Z_TOC LVL 1 = I.	 Appendix I. Selected C-Language Program Listings	91

@Z_TOC LVL 2 = I.1.	 Common Definitions and Variables	91

@Z_TOC LVL 2 = I.2.	 Clock<196>Filter Algorithm	92

@Z_TOC LVL 2 = I.3.	 Interval Intersection Algorithm	93

@Z_TOC LVL 2 = I.4.	 Clock<196>Selection Algorithm	94

@Z_TOC LVL 2 = I.5.	 Clock<196>Combining Procedure	95

@Z_TOC LVL 2 = I.6.	 Subroutine to Compute Synchronization Distance	96

@Z_TOC TITLE = List of Figures

@Z_TOC LVL 5 = Figure 1. Implementation Model	5

@Z_TOC LVL 5 = Figure 2. Calculating Delay and Offset	23

@Z_TOC LVL 5 = Figure 3. Clock Registers	37

@Z_TOC LVL 5 = Figure 4. NTP Message Header	46

@Z_TOC LVL 5 = Figure 5. NTP Control Message Header	50

@Z_TOC LVL 5 = Figure 6. Status Word Formats	51

@Z_TOC LVL 5 = Figure 7. Authenticator Format	59

@Z_TOC LVL 5 = Figure 8. Comparison of UTC and NTP Timescales at Leap	72

@Z_TOC LVL 5 = Figure 9. Network Time Protocol	73

@Z_TOC LVL 5 = Figure 10. NTP Phase-Lock Loop (PLL) Model	78

@Z_TOC LVL 5 = Figure 11. Timing Intervals	81

@Z_TOC LVL 5 = Figure 12. Measuring Delay and Offset	84

@Z_TOC LVL 5 = Figure 13. Error Accumulations	87

@Z_TOC LVL 5 = Figure 14. Confidence Intervals and Intersections	89

@Z_TOC TITLE = List of Tables

@Z_TOC LVL 5 = Table 1. System Variables	11

@Z_TOC LVL 5 = Table 2. Peer Variables	12

@Z_TOC LVL 5 = Table 3. Packet Variables	13

@Z_TOC LVL 5 = Table 4. Parameters	14

@Z_TOC LVL 5 = Table 5. Modes and Actions	21

@Z_TOC LVL 5 = Table 6. Clock Parameters	38

@Z_TOC LVL 5 = Table 7. Characteristics of Standard Oscillators	64

@Z_TOC LVL 5 = Table 8. Table of Leap-Second Insertions	70

@Z_TOC LVL 5 = Table 9. Notation Used in PLL Analysis	78

@Z_TOC LVL 5 = Table 10. PLL Parameters	80

@Z_TOC LVL 5 = Table 11. Notation Used in Error Analysis	83