@PARAFILTR ON =

@Z_TOC TITLE = Abstract

This report describes analytical models and design methodologies for computer clocks synchronized to other clocks in a computer network or internetwork. This requires each clock to function as a disciplined oscillator which can be adjusted in both time and frequency to compensate for the intrinsic errors of the onboard frequency source. It also requires the use of a time-synchronization protocol such as the Network Time Protocol, summarized briefly in this report, or the Digital Time Synchronization Service, described elsewhere.

The goal in this effort is to improve the synchronization accuracy from the tens of milliseconds regime of present timekeeping models to the submillisecond regime. In this report analytical models are used to express the accuracy and stability of a computer clock and to establish its design parameters with respect to time and frequency error tolerances. These models are based on the theory of adaptive-parameter, phase-lock loops. Included is a discussion on methods to combine the timing information received from a number of synchronized peer clocks to improve the accuracy and reliability of the local clock. Finally, a detailed analysis of the maximum and expected errors accumulated from the primary source of time throughout the synchronization subnet is presented.

Keywords: clock modelling, computer clock, synchronization error analysis, disciplined oscillator, synchronization algorithms, time synchronization, network synchronization.

Sponsored by: Defense Advanced Research Projects Agency under NASA Ames Research Center contract number NAG 2-638 and National Science Foundation grant number NCR-89-13623.

@PAGE =

@Z_TOC TITLE = Table of Contents

@Z_TOC LVL 2 = 1.	 Introduction	1

@Z_TOC LVL 2 = 2.	 Terms and Notation	1

@Z_TOC LVL 2 = 3.	 Network Time Protocol	3

@Z_TOC LVL 2 = 4.	 Computer Clock Models	5

@Z_TOC LVL 3 = 4.1.	 The Fuzzball Clock Model	7

@Z_TOC LVL 3 = 4.2.	 The Unix Clock Model	8

@Z_TOC LVL 3 = 4.3.	 A Precision Clock Model	9

@Z_TOC LVL 2 = 5.	 Mathematical Model of the Generic Local Clock	10

@Z_TOC LVL 3 = 5.1.	 Type-I Phase-Lock Loop	13

@Z_TOC LVL 3 = 5.2.	 Type-II Phase-Lock Loop	14

@Z_TOC LVL 3 = 5.3.	 The NTP Local-Clock Model	15

@Z_TOC LVL 3 = 5.4.	 Bandwidth Management	17

@Z_TOC LVL 2 = 6.	 Clock-Combining Algorithms	18

@Z_TOC LVL 2 = 7.	 Analysis of Errors	20

@Z_TOC LVL 3 = 7.1.	 Clock Modelling	22

@Z_TOC LVL 3 = 7.2.	 Measurement Errors	23

@Z_TOC LVL 3 = 7.3.	 Network Errors	25

@Z_TOC LVL 3 = 7.4.	 Inherited Errors	25

@Z_TOC LVL 2 = 8.	 Summary	28

@Z_TOC LVL 2 = 9.	 References	28

@Z_TOC TITLE = List of Figures

@Z_TOC LVL 6 = Figure 1. Subnet Synchronization Topologies	3

@Z_TOC LVL 6 = Figure 2. Network Time Protocol	3

@Z_TOC LVL 6 = Figure 3. Measuring Delay and Offset	4

@Z_TOC LVL 6 = Figure 4. Hardware Clock Models	5

@Z_TOC LVL 6 = Figure 5. Clock Adjustment Process	9

@Z_TOC LVL 6 = Figure 6. Phase-Lock Loop (PLL) Model	12

@Z_TOC LVL 6 = Figure 7. Timing Intervals	16

@Z_TOC LVL 6 = Figure 8. Allan Variance of WWVB Receiver	21

@Z_TOC LVL 6 = Figure 9. Error Accumulations	26

@Z_TOC TITLE = List of Tables

@Z_TOC LVL 7 = Table 1. Phase-Lock Loop Variables	11

@Z_TOC LVL 7 = Table 2. Phase-Lock Loop Parameters	11

@Z_TOC LVL 7 = Table 3. Variables Used in the NTP Local-Clock Model	15

@Z_TOC LVL 7 = Table 4. Notation Used in Error Analysis	22