@PARAFILTR ON =

@SUBHEAD = Abstract

This memo describes the Network Time Protocol (NTP) designed to distribute time information in a large, diverse internet system operating at speeds from mundane to lightwave. It uses a returnable-time architecture in which a distributed subnet of time servers operating in a self-organizing, hierarchical, master-slave configuration synchronizes local clocks within the subnet and to national time standards via wire or radio. The servers can also redistribute time information within a network via local routing algorithms and time daemons.

The architectures, algorithms and protocols which have evolved to NTP over several years of implementation and refinement are described in this paper. The synchronization subnet which has been in regular operation in the Internet for the last several years is described along with performance data which shows that timekeeping accuracy throughout most portions of the Internet can be ordinarily maintained to within a few tens of milliseconds, even in cases of failure or disruption of clocks, time servers or networks.

This memo describes the Network Time Protocol, which is specified as an Internet Standard in RFC-1119. Distribution of this memo is unlimited.

Keywords: network clock synchronization, standard time distribution, fault-tolerant architecture, maximum-likelihood estimation, disciplined oscillator, Internet protocol.

@PAGE =

@Z_TOC TITLE = Table of Contents

@Z_TOC LVL 1 = 1.	 Introduction	1

@Z_TOC LVL 2 = 1.1.	 Performance Requirements	2

@Z_TOC LVL 2 = 1.2.	 Discussion of Approaches	3

@Z_TOC LVL 1 = 2.	 Time Standards and Distribution	4

@Z_TOC LVL 1 = 3.	 Network Time Protocol	5

@Z_TOC LVL 2 = 3.1.	 Implementation Model	7

@Z_TOC LVL 3 = 3.1.1.	 Modes of Operation	7

@Z_TOC LVL 3 = 3.1.2.	 Data Formats	8

@Z_TOC LVL 3 = 3.1.3.	 State Variables	9

@Z_TOC LVL 2 = 3.2.	 Procedures	10

@Z_TOC LVL 2 = 3.3.	 Robustness Issues	11

@Z_TOC LVL 1 = 4.	 Sample Processing and Selection Operations	13

@Z_TOC LVL 2 = 4.1.	 Data Filtering	13

@Z_TOC LVL 2 = 4.2.	 Peer Selection	16

@Z_TOC LVL 1 = 5.	 Local Clock Design	18

@Z_TOC LVL 1 = 6.	 NTP in the Internet System	21

@Z_TOC LVL 2 = 6.1.	 Time Servers	21

@Z_TOC LVL 2 = 6.2.	 Synchronization Subnet	21

@Z_TOC LVL 2 = 6.3.	 Performance Analysis	22

@Z_TOC LVL 1 = 7.	 Future Directions	24

@Z_TOC LVL 1 = 8.	 References	25

@Z_TOC TITLE = List of Figures

@Z_TOC LVL 4 = Figure 1. Implementation Model	6

@Z_TOC LVL 4 = Figure 2. NTP Packet Header	9

@Z_TOC LVL 4 = Figure 3. Calculating Delay and Offset	11

@Z_TOC LVL 4 = Figure 4. Offset vs Delay	14

@Z_TOC LVL 4 = Figure 5. Phase-Lock Loop Model	18

@Z_TOC LVL 4 = Figure 6. Raw Offsets	22

@Z_TOC LVL 4 = Figure 7. Filtered Offsets	23

@Z_TOC LVL 4 = Figure 8. Error Distribution	23

@Z_TOC TITLE = List of Tables

@Z_TOC LVL 4 = Table 1. Outlyer Selection Procedure	17