News Flash

Newark, DE, 28 April 1999

The University of Delaware announced significant progress in computing science today. Prof. David Mills, working under contract for the Defense Advanced Projects Agency, described schemes where communicating processes, in this case a corps of network timekeepers, could be dropped behind enemy lines without detection and where each timekeeper is dropped by random parachute to an unknown coordinate. Using innovative multicasting technology layered on a conventional or wireless network, the timekeeper lieutenants discover each other and automatically organize themselves as a tree of servers and clients saluting a cadre of previously appointed timekeeper generals. However, the number of lieutenants permitted at each command level (stratum) is limited to  avoid compromise, should the network become overloaded, and the distance between the radios (synchronization distance) is limited to control accuracy.

Mills described three schemes required for successful deployment of the timecasting corps. The first is a cryptographic authentication scheme called autokey, which uses an backwards-predicable pseudo-random sequence of keys so each client can authenticate the current message using a key computed as a deterministic hash of the message contents and the previous key. The sequence is cryptographically bound to public values obtained from the DNS and the server over unsecure facilities. This scheme was described at a previous DARPA PI meeting and in a set of status reports and briefing slides on the web at www.eecis.udel.edu/~mills/autokey.htm.

The second scheme is an automated discovery/configuration algorithm which uses multicast/anycast techniques to discover new servers and clients and automatically organize the timecasting corps to minimize the synchronization distance while observing constraints of limited fanout and maximum distance. A vital ingredient to the success of this technique is the ability of the NTP algorithms to rank the servers by metric and automatically add/drop selected servers to maximize the overall quality.This scheme was described at a previous DARPA PI meeting and in a set of status reports and briefing slides on the web at www.eecis.udel.edu/~mills/autocfg.htm.

The third scheme is a set of crafted kernel modifications to the Unix kernel which inherently resists attacks which try to destabilize the kernel clock. These modifications work in conjunction with the NTP algorithms which select the best candidates from among a set of servers using Byzantine Agreement principles augmented by engineered statistical filters. The modifications were described at the recent DARPA High Confidence Networks meeting and in a set of briefing slides at www.eecis.udel.edu/~mills/colloq.htm. The modifications have been incorporated in SunOS, Digital Unix, Linux and FreeBSD. The software distribution can be retrieved via the web or ftp from www.eecis.udel.edu in pub/ntp/nanokernel.tar.Z.

Mills further elaborated on the current status and future plans in each of these schemes and related projects in the form of briefing slides in PostScript, HTML, PowerPoint and PDF formats at www.eecis.udel.edu/~mills/status.htm. Links from these pages reveal current and previous quarterly reports, yearly reports, final reports quad charts for the current and previous funding cycles.

-30-

Dave sends